| Date: | 13/04/2026 | Deviation No. | DEV/PR/25/008 |
| --- | --- | --- | --- |
| Investigation tool used: 6M 5 Why Brainstorming |  |  |  |
| Other Tools (If any): |  |  |  |
| Not applicable |  |  |  |
| Define: |  |  |  |
| Following checks shall be considered while writing the “Define” section. - Clearly define what happens actually. - Explain what is different than expected. - Mention the location where the deviation has occurred. - Date/time of deviation occurrence and date/time of detection. - Mention the name of personnel who is involved in the deviation. - Mention initial scope of deviation (i.e., impacted product/Material/Equipment/System/Batches/etc.) On 28/03/2026 at 18:39 hrs, in the Depyrogenating Tunnel (Equipment ID: E/PR/069) located in the Washing and Depyrogenation Room (GF-61), the production operator (Emp. ID: 1077) inadvertently initiated the Sterilizing Chamber Filter Conditioning (SFC) mode instead of the intended Cooling Chamber Sterilization (CCS) mode. As per SOP/DP/PR/041 (Sr.No. 7.3.3.5), the SFC mode is required to be selected only during installation of new filters. However, in this instance, SFC mode was selected during routine operation instead of CCS mode, which is a deviation from the approved procedure. The deviation occurred on 28/03/2026 at 18:39 hrs and was identified on 29/03/2026 at 01:06 hrs after completion of the cycle. The initial scope of the deviation includes the Depyrogenating Tunnel (Equipment ID: E/PR/069) and the specific cycle executed during the mentioned period. |  |  |  |
| Details Investigation: |  |  |  |
| Measure: Following checks shall be considered while writing the “Measure” section. - Does the summary provide relevant facts and data/information that was reviewed including: environment, process/product history, personnel info (title and job title), controls limits, etc. - Is a summary of the analysis of the factors and data provided? - Is a conclusion statement of the analysis and review provided? - If there were Regulatory Notification, were details provided? - Is the report written in a logical flow and easily understood by the reader? The Depyrogenating Tunnel (Equipment ID: E/PR/069) located in the Washing and Depyrogenation Room (GF-61) is used for sterilization and depyrogenation of containers prior to aseptic processing. The system operates under validated parameters to ensure sterilization efficiency, airflow control, and maintenance of environmental integrity. On 28/03/2026, equipment cleaning was performed from 03:34 hrs to 04:50 hrs as per the approved procedure. Post-cleaning verification confirmed that the equipment was clean, dry, and fit for operation, with no abnormalities observed. At 18:39 hrs on 28/03/2026, the production operator (Emp. ID: 1077), initiated a cycle intended for Cooling Chamber Sterilization (CCS). However, due to inadvertent human error, the Sterilizing Chamber Filter Conditioning (SFC) cycle was selected. The cycle was executed and completed without interruption on 29/03/2026 at 01:06 hrs, with a total run time of approximately 06 hours and 26 minutes. The cycle was ended by another Production Operator (Emp. ID: 842), who identified the non-conformance. A detailed review of the unintended SFC cycle data was performed. All critical process parameters were found to be within validated and acceptable limits, indicating that the equipment functioned as intended during the SFC cycle. SFC Cycle Parameter: Description; Unit; Value. TU-SFC sterilizing chamber air temperature-TE22; °C; 310. TU-SFC cooling chamber air temperature-TE32; °C; 20. TU-SFC inlet chamber air speed- PT11; m/s; 0.55. TU-SFC sterilizing chamber air speed- PT21; m/s; 0.75. TU-SFC cooling chamber air speed- PT31; m/s; 0.75. TU-SFC inlet chamber /Env.dP-PT50; Pa; 13. TU-SFC cooling chamber /Env.dP-PT51; Pa; 15. TU-SFC heating temperature increase; °C; 1.5. TU-SFC conditioning time; Min; 20. TU-SFC heating temperature decrease; °C/min; 1.5. TU-SFC controlled temperature decrease limit; °C; 180. The above data confirms that all SFC cycle parameters were achieved within predefined specifications. No critical alarms, excursions, or abnormal trends were observed during the cycle, confirming stable equipment performance. Following identification of the incorrect cycle, the intended CCS cycle was initiated on 29/03/2026 at 02:25 hrs by Production Operator (Emp. ID: 842). The cycle was completed successfully at 08:17 hrs, with a total duration of approximately 05 hours and 52 minutes. The CCS cycle report was reviewed, and all critical parameters were found to be within validated limits. CCS Cycle Parameter: Description; Unit; Value. TU-CCS sterilizing chamber air temperature-TE22; °C; 160. TU-CCS cooling chamber air temperature-TE32; °C; 210. TU-CCS air tight door temperature- TE34; °C; 220. TU-CCS inlet chamber air speed-PT11; m/s; 0.5. TU-CCS sterilizing chamber air speed-PT21; m/s; 0.65. TU-CCS cooling chamber air speed-PT31; m/s; 0.65. TU-CCS cooling chamber plenum dP-PT61; Pa; -2. TU-CCS sterilization heating temperature increase; °C/min; 1.5. TU-CCS cooling heating temperature increase; °C/min; 1.5. TU-CCS sterilization time; min; 12. TU-CCS 4th door position; mm; 90. The CCS cycle met all predefined acceptance criteria, with no deviations or alarms recorded, confirming successful completion of the intended sterilization process. A review of historical data confirmed that the sterilizing chamber typically operates at approximately 320°C during routine depyrogenation cycles for vial format. The observed temperature during both SFC and CCS cycles were within validated operational ranges, demonstrating consistent equipment performance. Post-deviation verification of the HEPA filter air velocity and Non-Viable Particle Count (NVPC) monitoring performed. All results were found to be within specified limits, confirming that the filter and environmental controls remained unaffected. Refer below table for more details, Air Velocity Results Sr. No.; Equipment Name; Filter Code; Average (m/s). 1; Depyrogenating Tunnel (E/PR/069); DF11; 0.57. DF 21; 0.76. DF 22; 0.78. DF 31; 0.76. DF 32; 0.77. NVPC Results Sr. No.; Equipment Name; Filter Code; 0.5 micron; 5.0 micron. 1; Depyrogenating Tunnel (E/PR/069); L1; 767; 9. L2; 190; 13. L3; 181; 7. L4; 6; 0. L5; 10; 1. The production operator (Emp. ID: 1077) is trained, qualified, and experienced in operating the depyrogenating tunnel, including execution of CCS cycles. No previous deviations or errors associated with the operator were identified, indicating that the event was an isolated human error. Awareness training was provided to all concerned personnel adherence to SOP requirements and correct cycle selection. Environmental monitoring data for the Washing and Depyrogenation Room (GF-61) during the activity period were reviewed and found to be within specified limits. Based on the comprehensive evaluation of process data, equipment performance, environmental conditions, and personnel qualification, it is concluded that the unintended execution of the SFC cycle did not adversely impact system performance. The subsequent CCS cycle was successfully completed and met all acceptance criteria. Based on the above evaluation, there is no impact on product quality or patient safety is anticipated. As a result, regulatory notification is not required. |
| Analyze: Various investigation tools used to analyze the root cause like 6M, 5 Why, Personnel interview, Brainstorming etc. |  |  |  |
| 6 M Method (If Applicable): - Man: Not Applicable. - Machine: Not Applicable. - Measurement: Not Applicable. - Material: Not Applicable. - Method: Not Applicable. - Milieu (Environment): Not Applicable. Conclusion Not Applicable |  |  |  |
| Note: No 6 M question shall be deleted in the investigation, If any question is not applicable, then mention in ans. as “Not Applicable”. |  |  |  |
| 5 Why Approach (If Applicable): 1. WHY: Why was the SFC cycle initiated instead of CCS cycle? Ans. Because the production operator (Emp. ID: 1077) selected the incorrect cycle (SFC) on the HMI during operation. 2. WHY: Why did the operator select the incorrect cycle on the HMI? Ans. Because the operator did not adequately verify the selected cycle before initiating the process. 3. WHY: Why was the verification of cycle selection not adequately performed? Ans. Because the step of cycle verification before initiation was not strictly followed as per SOP requirements. 4. WHY: Why was the cycle start verification step not strictly adhered to? Ans. Because the activity was routine in nature, which led to a momentary lapse in attention during execution, resulting in the SOP-defined verification step not being performed as required. 5. WHY: Why did the lapse in attention occur during a routine activity? Ans. Because of individual human oversight, despite the operator being trained, qualified, and the procedure clearly defines the required verification step Conclusion: Based on the above analysis, it is concluded that, the deviation occurred due to an isolated human error, wherein the operator did not adhere to the SOP-defined verification step prior to cycle initiation. The procedure adequately defines the required controls, and the operator is trained and experienced. No deficiencies in procedure, system, or equipment were identified. |  |  |  |
| Brainstorming: Not Applicable |  |  |  |
| Other Tool if Any: Not Applicable |  |  |  |
| Other Tool if Any: Not Applicable |  |  |  |
| Investigation Outcome: |  |  |  |
| - The investigation was performed using DMAIC methodology and 5-Why analysis. - Based on the detailed investigation, it was determined that the deviation occurred due to inadvertent human error, wherein the production operator (Emp. ID: 1077) selected the Sterilizing Chamber Filter Conditioning (SFC) cycle instead of the intended Cooling Chamber Sterilization (CCS) cycle on the HMI. - This occurred due to inadequate verification of cycle selection prior to initiation, indicating a lapse in adherence to the defined procedural requirement. - An evaluation of the equipment performance was performed by reviewing the cycle data of the executed SFC cycle. All critical process parameters, including temperature, airflow velocity, differential pressure, and cycle duration, were found to be within validated and predefined acceptance limits. No critical alarms, excursions, or abnormal trends were observed during the cycle, confirming that the equipment functioned as intended. - Following identification of the deviation, the intended CCS cycle was executed. Review of the CCS cycle data confirmed that all critical parameters met the predefined acceptance criteria, and the cycle was completed successfully without any deviations or alarms, thereby ensuring that the required sterilization conditions were achieved. - Further assessment included review of environmental monitoring data and post-deviation verification checks such as HEPA filter air velocity and Non-Viable Particle Count (NVPC). All results were found to be within specified limits, confirming that environmental conditions and filter efficiency remained unaffected. - Personnel evaluation confirmed that the involved operator is trained, qualified, and experienced in operating the depyrogenating tunnel, including execution of CCS cycles. A review of historical records indicated no similar deviations associated with the operator, supporting that this incident is an isolated human error. - The investigation confirmed that the procedure adequately defines the required verification steps, and no deficiencies in procedure, equipment, or system design were identified. - Based on the overall assessment of process data, equipment performance, environmental conditions, and personnel qualification, it is concluded that the unintended execution of the SFC cycle did not adversely impact system performance or process integrity. The subsequent successful execution of the CCS cycle ensured compliance with sterilization requirements. - Therefore, no impact on product quality or patient safety is anticipated, and the event is categorized as an isolated human error due to non-adherence to an established procedural requirement. Based on the nature of non-conformance, a chronology of event, and detailed investigation, further root cause categorization has been done as per format for “Root Cause Categorization” SOP/DP/QA/008-F04 and root cause for the non-conformance was identified. |  |  |  |
| Identified Root Cause/ Probable Cause: |  |  |  |
| Based on the detailed investigation, the two most probable root cause as mentioned below , - Primary Root Cause Level 1: Human Performance - Secondary Root Cause Level 2: Omission error. - Third Root Cause Level 3: Responsibility boundaries ineffective. |  |  |  |
| Impact Assessment (System/ Document/ Product/ Equipment/Patient safety/Past batches): |  |  |  |
| System: The deviation was related to production operator (Emp. ID: 1077) selected the Sterilizing Chamber Filter Conditioning (SFC) cycle instead of the intended Cooling Chamber Sterilization (CCS) cycle on the HMI. The depyrogenating tunnel operated within validated parameters during both the unintended SFC cycle and the subsequent CCS cycle. The system functioned as per its intended use. Hence, there is no impact on system. Document: The deviation was related to production operator (Emp. ID: 1077) selected the Sterilizing Chamber Filter Conditioning (SFC) cycle instead of the intended Cooling Chamber Sterilization (CCS) cycle on the HMI. The deviation is attributed to non-adherence to an existing SOP requirement, and no revision of documents is required. Product: The deviation was related to production operator (Emp. ID: 1077) selected the Sterilizing Chamber Filter Conditioning (SFC) cycle instead of the intended Cooling Chamber Sterilization (CCS) cycle on the HMI. The unintended SFC cycle did not involve product exposure , and the subsequently executed CCS cycle met all sterilization acceptance criteria. Hence, there is no impact on product Equipment: The deviation was related to production operator (Emp. ID: 1077) selected the Sterilizing Chamber Filter Conditioning (SFC) cycle instead of the intended Cooling Chamber Sterilization (CCS) cycle on the HMI. The deviation is attributed to non-adherence to an existing SOP requirement. Due to occurred non conformance No impact on equipment performance. The intended CCS cycle confirm that the equipment function as per its intended use. Patient safety / Past Batches: The deviation was related to production operator (Emp. ID: 1077) selected the Sterilizing Chamber Filter Conditioning (SFC) cycle instead of the intended Cooling Chamber Sterilization (CCS) cycle on the HMI. Post deviation executed intended CCS cycle verified and found satisfactory. The deviation is related to departure from approved procedure and no similar incidents were identified in past batch records. |  |  |  |
| Improve: Improve section covers the corrective actions Following checkpoint shall be considered as guidance only while finalizing the corrective actions, - Is a specific corrective Actions identified (including applicable Immediate Actions) to remediate the current issue or situation such that the associated system was returned to a state of control/compliance and also it will avoid the recurrence of deviation? - Is specific Corrective Actions identified for each root cause /substantiated probable root cause, as applicable? - Is the Corrective action assigned a unique number, responsible person and due date so it can be tracked? Does the action describe what will be the expected outcome they can be verified? - Are the identified corrective actions achievable based on the information provided? |  |  |  |
| Corrective Action: |  |  |  |
| Upon identification of the non-conformance following corrective actions has been taken, - Load cell calibration was re-performed against Work Order No. WO/PR/26-012. - Re-Calibration report generated with correct data capture and verified. - Awareness training provided to concerned operators. Further, this is an isolated case which has been occurred for the first time due to which there is no any requirement of additional corrective action as non-conformance has occurred due to oversight error. |  |  |  |
| Control: Control section covers the preventive actions Following checkpoint shall be considered while finalizing the preventive actions, - Were specific Preventive Actions identified for each root cause /substantiated probable root cause as applicable? - Was the Preventive Action linked the classification of the root cause and explanation given for how it will prevent recurrence? - Was the Preventive action assigned a unique number, responsible person and due date so it can be tracked? Does the action describe and expected outcome can be verified? - Was Effectiveness Verification required or not and the rationale for either documented based on the quality impact of the incident after the investigation concluded? - Was an Interim Plan needed to ensure a state the control while the Preventive Actions were implemented? - Was rationale provided when no Preventive Action were identified? - Does the Final Comments section include rotational to support the conclusion of the investigation and CAPA. - Was each of the impact assessment fields completed correctly - Regulatory Impact, Regulatory notification Product Quality, Validation, Stability, Market/Clinical? - Does the Recommended Lot disposition match the conclusions of the investigation and Impact assessment? - Does the Conclusion include final decision and rationale other regulatory notification required? - CAPA required to release material or batches have been verified to be complete and closed prior to material or batch disposition. Any related CAPA that remain open, but have no impact on the material or batch release and a documented explanation is provided. - Does the Conclusion include a brief summary of root cause, final scope/impact including rationale for product/material disposition, impact assessment and relevant lot/material/equipment details? - Are the identified preventive actions achievable based on the information provided? |  |  |  |
| Preventive Action: |  |  |  |
| The observed non-conformance is attributed to isolated human error (oversight), as the procedure adequately defines the required verification steps, and no deficiencies in the procedure, equipment, or system design were identified. Furthermore, this is a first-time occurrence with no prior history of similar events. Therefore, no standalone Corrective and Preventive Action (CAPA) is proposed. Interim control is not required because: - The procedure adequately defines the required controls, including verification of cycle selection prior to initiation. - The operator involved is trained, qualified, and experienced, with no history of similar errors. - No deficiencies in system, equipment, or procedure were identified. - The equipment operated within validated parameters, and all environmental and performance checks were satisfactory. - The subsequent CCS cycle was successfully executed, ensuring compliance with sterilization requirements. |  |  |  |
| Document Reviewed: |  |  |  |
| - Batch manufacturing Record (B. No. B092542503) - Calibration report - Work order (WO/PR/26-007) - Training record |  |  |  |
| List of attachments (If applicable): |  |  |  |
| Attachment No. I: Photocopy of BMR Attachment No. II: Photocopy of calibration report Attachment No. III: Photocopy of recalibration report Attachment No. IV: Photocopy of work order Attachment No. V: Awareness training record Attachment No. VI: Root cause categorization Attachment No. VII: Personnel interview Attachment No. VIII: Results of HEPA filter air velocity and Non-Viable Particle Count |  |  |  |

| Prepared By (Sign/ Date) | Reviewed By (Sign/ Date) | Reviewed By (Sign/ Date) | Reviewed By (Sign/ Date) | Reviewed By (Sign/Date) | Reviewed By (Sign/ Date) | Approved By QA (Sign/ Date) |
| --- | --- | --- | --- | --- | --- | --- |