| Date: | 25/11/2025 | Deviation No. | DEV/PR/25/004 |
| --- | --- | --- | --- |
| Investigation tool used: 6M 5 Why Brainstorming |  |  |  |
| Other Tools (If any): |  |  |  |
| Not applicable |  |  |  |
| Define: |  |  |  |
| Following checks shall be considered while writing the “Define” section. - Clearly define what happens actually. - Explain what is different than expected. - Mention the location where the deviation has occurred. - Date/time of deviation occurrence and date/time of detection. - Mention the name of personnel who is involved in the deviation. - Mention initial scope of deviation (i.e., impacted product/Material/Equipment/System/Batches/etc.) On 20/11/2025 at approximately 11:11 hrs, a temperature overshoot (125.2 °C ) was identified by a production colleague (Employee ID: 1036) during the heat-up phase of the PPM load (Cycle No. 3525) for Insulin Injection Biphasic Isophane Ph. Eur./IP 100 IU/mL, 3 mL Cartridge (Batch No. B042527518). The maximum temperature recorded was 125.7°C, and the overshoot persisted for approximately 1 hour and 24 minutes. The event occurred in the HPHV Steam Sterilizer (Equipment ID: E/PR/019) located in the autoclave room (GF-73). The temperature overshoot observed specifically on the T4 temperature sensor, located at the right-top corner side of the unloading area. As per SOP/DP/PR/018 – “Operation and Cleaning of HPHV Steam Sterilizer”, Section 7.6.6.5, the required sterilization temperature range is 121.1°C to 124.0°C. However, during the heat-up phase, the temperature exceeded the upper limit, reaching 125.7°C. Additionally, the cycle remained stuck in the heat-up phase for approximately 1 hour and 24 minutes, after which it was manually aborted. Further, a “Chamber Condensate Temperature Sensor 1 Fail” alarm was triggered at 11:46:23 hrs on 20/11/2025, lasting for 3 seconds. Following identification of the non-conformance, Breakdown No. BD/PR/25-018 was raised for rectification. Subsequently, Deviation No. DEV/PR/25/004 was initiated on 20/11/2025 at 12:44 hrs to investigate and identify the root cause. |  |  |  |
| Details Investigation: |  |  |  |
| Measure: Following checks shall be considered while writing the “Measure” section. - Does the summary provide relevant facts and data/information that was reviewed including: environment, process/product history, personnel info (title and job title), controls limits, etc. - Is a summary of the analysis of the factors and data provided? - Is a conclusion statement of the analysis and review provided? - If there were Regulatory Notification, were details provided? - Is the report written in a logical flow and easily understood by the reader? The HPHV Steam Sterilizer (Equipment ID: E/PR/019) is used for sterilizing materials entering the aseptic processing area. On 20/11/2025, the following sequence of activities was carried out in the autoclave. Sr. No.; Date; Time in Hrs.; Activity. 1; 20/11/2025; 01:07; Chamber cleaning performed. 2; 02:28; Hot leak test performed to ensure the chamber integrity. 3; 03:09; Bowie dick test performed to ensure air removed properly. 4; 06:15; Garment Load. 5; 08:50; CTZ Machine Parts 1 load for Insulin Injection Biphasic Isophane 100 IU/mL, 3 ml cartridge (Batch No. B042527518). 6; 12:35; PPM load for Insulin Injection Biphasic Isophane 100 IU/mL, 3 ml cartridge (Batch No. B042527518).. During the PPM Load ( Cycle No. 3515), a temperature overshoot of 125.2 °C was observed at 11:11 hrs. in heat up phase. The maximum temperature recorded was 125.7°C, and the overshoot temperature persisted for approximately 1 hour and 24 minutes in heat up phase. The cycle remained stuck in the heat up phase and was manually aborted. As per SOP/DP/PR/018-R04, the sterilization temperature range must be maintained between 121.1°C and 124.0°C. Refer below table for more details of cycle no. 3515, Sr. No.; Sensor No.; Location; Maximum temperature in ºC; Maximum pressure in Bar; Heat up phase Start time; Heat up phase End time; Total Duration. 1; T1; Chamber Drain; 105.2; 0.252; 11:11:39; 12:35:10; 01 hr. 24 minute. 2; T2; Loading left top side; 122.3. 3; T3; Unloading left bottom side; 125.2. 4; T4; Unloading right top side; 125.7. 5; T5; Loading right bottom side; 120.6. Also, chamber condensate temperature sensor 1 fail alarm was triggered at 11:46:03 hrs. for 03 seconds. As an immediate action, the cycle was manually aborted in the heat-up phase, and Breakdown No. BD/PR/25-018 was raised at 12:50 hrs on 20/11/2025 for rectification. A deviation was raised at 16:07 hrs on 20/11/2025 to investigate the event and identify the root cause. An engineering colleague (Employee ID: 921) attended the breakdown as per the breakdown request. During the investigation, it was observed that during the heat-up phase, drain sensor T1 remained at 105.2°C for an extended duration and failed to reach the sterilization initiation threshold of 121.7°C, while the remaining sensors had already achieved the required temperature. As the other sensors reached the required temperature, the system’s control logic stopped the incoming pure steam supply to the chamber. Due to the stop of pure steam, the chamber pressure did not increase to the required sterilization pressure range of 1.03 to 1.3 bar, preventing the drain sensor (T1) from further increasing in temperature. Consequently, the cycle remained stuck in the heat-up phase for approximately 1 hour and 24 minutes. During the breakdown attempt, the Engineering colleague (Employee ID: 921) observed that the jacket pressure had increased up to 2.8 bar. Engineering colleague (Employee ID: 921) inspected the jacket steam inlet valve and found that the valve was not functioning as intended, which resulted in continuous raw steam entering the jacket. This led to an abnormal increase in jacket pressure (up to 2.8 bar), which in turn caused an increase in chamber temperature readings, even though the chamber pressure remained low. Since the chamber condensate temperature sensor (T1) is located at the drain point, the increased jacket steam pressure did not restrain the drain sensor (T1), and therefore its temperature remained low. As a result, drain sensor T1 did not reach the required temperature, and the cycle remained stuck in the heat-up phase. Additionally, a loose wiring connection was identified for Chamber Condensate Temperature Sensor-1, due to which sensor fail alarm triggered for 3 seconds. The Engineering college (Employee Id: 868) tightened the sensor wiring and rectified the jacket steam inlet valve. Following the repairs, cleaning activity performed and a trial bowie dick test (cycle No. 3516) taken. During trial cycle (3516) temperature overshoot alarm triggered and cycle aborted as jacket pressure was high. Therefore, jacket pressure adjusted and again a trial bowie dick test (cycle no. 3517) taken and found satisfactory. After breakdown rectification by EU, a routine Hot Leak Test (Cycle No. 3520) and Bowie Dick Test (Cycle No. 3521) were performed, and both cycles were found satisfactory. Based on the assessment of the Engineering colleague (Employee ID: 921), the most probable root cause was the malfunctioning of the jacket steam inlet valve (Valve No. 101), which resulted in the cycle remaining stuck in the heat-up phase and ultimately being aborted manually. Regarding material disposition, additional PPM (Plunger stopper and combi seals) were issued and used for batch activities, while the exposed PPM from Cycle No. 3515 was discarded and will be used only for qualification activities. However, the exposed glass beads from Cycle No. 3515 were used for the batch, as there is no impact from repeated sterilization up to three cycles, as per vendor recommendations as the glass beads are made of USP Class VI compliant material. Based on the above evaluation, no impact on product quality or patient safety is anticipated, and therefore regulatory notification is not required. |
| Analyze: Various investigation tools used to analyze the root cause like 6M, 5 Why, Personnel interview, Brainstorming etc. |  |  |  |
| 6 M Method (If Applicable): - Man: Not Applicable. - Machine: Not Applicable. - Measurement: Not Applicable. - Material: Not Applicable. - Method: Not Applicable. - Milieu (Environment): Not Applicable. Conclusion Not Applicable |  |  |  |
| Note: No 6 M question shall be deleted in the investigation, If any question is not applicable, then mention in ans. as “Not Applicable”. |  |  |  |
| 5 Why Approach (If Applicable): 1. WHY: Why was the cycle aborted manually in heat up phase? Ans. The cycle was aborted manually in heat up phase because, it remained stuck in heat up phase for 01 hr. 24 minutes. 2. WHY: Why did the cycle remain stuck in the heat up phase for 01 hr. 24 minutes ? Ans. The cycle remained stuck in heat up phase because drain sensor (T1) did not reach the required sterilization temperature of 121.1 ºC . 3. WHY: Why did drain sensor T1 not reach the required temperature of 121.1 ºC? Ans. Drain sensor T1 did not reach the required temperature because chamber pressure was low ( maximum observed pressure: 0.252 Bar). 4. WHY: Why was the chamber pressure low? Ans. The chamber pressure was low, because incoming pure steam in chamber had stopped. 5. WHY: Why did incoming pure steam to the chamber stop? Ans. Pure steam was stopped, because all other chamber sensors (T2,T3,T4,T5) had already reached sterilization temperature causing control logic to stop pure steam supply despite low chamber pressure. 6. WHY: Why did all other chamber sensors reach sterilization temperature despite low chamber pressure? Ans. All other sensors reached sterilization temperature because the jacket pressure increased up to 2.8 bar, which influenced chamber temperature readings. 7. WHY: Why did the jacket pressure increase up to 2.8 bar ? Ans. The jacket pressure increased because raw steam was continuously entering the jacket. 8.WHY: Why was raw steam continuously entering the jacket? Ans. Raw steam continuously entered the jacket because the jacket steam inlet valve (Valve 101) was malfunctioning. Conclusion: Based on the 5-Why analysis, the deviation occurred due to a malfunctioning jacket steam inlet valve (Valve 101), which allowed continuous raw steam entry into the jacket, causing jacket pressure to rise to 2.8 bar. This led to an artificial increase in chamber temperature (up to 125.7°C) despite low chamber pressure, resulting in the cycle becoming stuck in the heat-up phase, and ultimately requiring manual abort. Additionally, it was identified that no routine operational verification frequency available for the jacket steam inlet valve (Valve 101). |  |  |  |
| Brainstorming: Not Applicable |  |  |  |
| Other Tool if Any: Not Applicable |  |  |  |
| Other Tool if Any: Not Applicable |  |  |  |
| Investigation Outcome: |  |  |  |
| The investigation was conducted using the DMAIC and 5-Why methodologies. Since the deviation was categorized as “minor”, a preliminary investigation was not required. The nonconformance occurred due to a temperature overshoot during the heat-up phase of the PPM load (Cycle No. 3515) for Insulin Injection Biphasic Isophane 100 IU/mL, 3 mL cartridge (Batch No. B042527518). The maximum temperature recorded was 125.7°C, lasting for approximately 01 hour 24 minutes. To identify the root cause, a breakdown request (BD/PR/25-018) was raised. During the breakdown rectification, the engineering colleague (Employee ID: 921) identified that the jacket steam inlet valve (101) was malfunctioning, resulting in the jacket pressure rising to 2.8 bar. Due to the increased jacket pressure, the chamber temperature sensor reached up to 125.7°C, causing the cycle to remain stuck in the heat-up phase for about 01 hour 24 minutes, after which the cycle was manually aborted. The PPM (plunger stopper and combi seal) exposed in Cycle No. 3515 was discarded, except for the glass beads. Additional PPM (plunger stopper and combi seal) along with the exposed glass beads were used for batch activities. As per vendor recommendations, glass beads can be used for up to three repeat cycles, therefore no impact is anticipated from their reuse. Additionally, it was noted that no defined regular frequency exists for the verification of jacket steam inlet valve (101). Based on the nature of non-conformance, a chronology of event, and detailed investigation, further root cause categorization has been done as per format for “Root Cause Categorization” SOP/DP/QA/008-F04 and root cause for the non-conformance was identified as ‘Equipment/Instrument’. |  |  |  |
| Identified Root Cause/ Probable Cause: |  |  |  |
| Based on the detailed investigation, the most probable root cause as mentioned below - Primary Root Cause Level 1: Equipment/Instrument. 2. Secondary Root Cause Level 2: Mechanical failure. 3. Third Root Cause Level 3: Not Applicable. |  |  |  |
| Impact Assessment (System/ Document/ Product/ Equipment/Patient safety/Past batches): |  |  |  |
| System: The nonconformance pertains to temperature overshoot during PPM load (Cycle no. 3515) lasting for approximately 01 Hr. 24 minutes. The breakdown form BD/PR/25-018 was raised to rectify the issue. Against breakdown, the issue rectified and trial taken and found satisfactory. The system is operating as per its intended use. Hence does not anticipate impact on the system. Document: The nonconformance pertains to temperature overshoot during PPM load (Cycle no. 3515) lasting for approximately 01 Hr. 24 minutes. The nonconformance occurred due to malfunctioning of jasket steam inlet valve (101) as there is no frequency defined for operation verification of valve. Therefore, PM checklist shall be revised to incorporate the operation verification jacket steam inlet valve (101) Product: The nonconformance is related to temperature overshoot occurred in PPM load (Cycle no. 3515) lasting for approximately 01 Hr. 24 minutes. Against deviation, additional PPM (Plunger stoppers & combi seals) were used for batch activities. Therefore, no impact on product quality is anticipated. Hence, there is no impact on products. Equipment: The nonconformance is related to temperature overshoot occurred in PPM load (Cycle no. 3515) lasting for approximately 01 Hr. 24 minutes. The breakdown no. BD/PR/25-018 was raised to rectify the issue. Against breakdown, the issue rectified and trial taken and found satisfactory. The equipment is operating as per its intended use. Hence does not anticipate impact on the equipment Patient safety / Past Batches: The nonconformance is related to temperature overshoot occurred in PPM load (Cycle no. 3515) lasting for approximately 01 Hr. 24 minutes. Against deviation, additional PPM issued and used for batch activities. Hence, it does not have an impact on patient safety/past batches. |  |  |  |
| Improve: Improve section covers the corrective actions Following checkpoint shall be considered as guidance only while finalizing the corrective actions, - Is a specific corrective Actions identified (including applicable Immediate Actions) to remediate the current issue or situation such that the associated system was returned to a state of control/compliance and also it will avoid the recurrence of deviation? - Is specific Corrective Actions identified for each root cause /substantiated probable root cause, as applicable? - Is the Corrective action assigned a unique number, responsible person and due date so it can be tracked? Does the action describe what will be the expected outcome they can be verified? - Are the identified corrective actions achievable based on the information provided? |  |  |  |
| Corrective Action: |  |  |  |
| After identified of nonconformance, information given to production head and QA head and deviation DEV/PR/25/002 is initiated. As an immediate action breakdown no. BD/PR/25-018 raised to rectify the issue. Against breakdown, the issue rectifies and trial VLT (Cycle no. 3520 ) and bowie dick ( cycle no. 3521) taken and found satisfactory. Also, the equipment works as per its intended use. Further, this is an isolated case which has been occurred for the first time due to which there is any no requirement of Corrective Action. |  |  |  |
| Control: Control section covers the preventive actions Following checkpoint shall be considered while finalizing the preventive actions, - Were specific Preventive Actions identified for each root cause /substantiated probable root cause as applicable? - Was the Preventive Action linked the classification of the root cause and explanation given for how it will prevent recurrence? - Was the Preventive action assigned a unique number, responsible person and due date so it can be tracked? Does the action describe and expected outcome can be verified? - Was Effectiveness Verification required or not and the rationale for either documented based on the quality impact of the incident after the investigation concluded? - Was an Interim Plan needed to ensure a state the control while the Preventive Actions were implemented? - Was rationale provided when no Preventive Action were identified? - Does the Final Comments section include rotational to support the conclusion of the investigation and CAPA. - Was each of the impact assessment fields completed correctly - Regulatory Impact, Regulatory notification Product Quality, Validation, Stability, Market/Clinical? - Does the Recommended Lot disposition match the conclusions of the investigation and Impact assessment? - Does the Conclusion include final decision and rationale other regulatory notification required? - CAPA required to release material or batches have been verified to be complete and closed prior to material or batch disposition. Any related CAPA that remain open, but have no impact on the material or batch release and a documented explanation is provided. - Does the Conclusion include a brief summary of root cause, final scope/impact including rationale for product/material disposition, impact assessment and relevant lot/material/equipment details? - Are the identified preventive actions achievable based on the information provided? |  |  |  |
| Preventive Action: |  |  |  |
| Following preventive actions proposed based on the outcome of the investigation. - PM checklist shall be revised to include operation verification of jacket steam inlet valve (101) during PM. Preventive action is related to the revision of PM checklist, hence interim control is not required. |  |  |  |
| Document Reviewed: |  |  |  |
| - SOP/DP/PR/018-R03 (Operation and cleaning of HPHV Steam sterilizer). - Process printouts. - PM check list. |  |  |  |
| List of attachment (If applicable): |  |  |  |
| Attachment No. I: Photocopy of Breakdown. Attachment No. II: Photocopy of Cycle report. Attachment No. III: Trial cycle report. Attachment No. IV: Impact assessment. |  |  |  |

| Prepared By (Sign/Date) | Reviewed By (Sign/Date) | Reviewed By (Sign/Date) | Reviewed By (Sign/Date) | Reviewed By (Sign/Date) | Reviewed By (Sign/Date | Approved By QA (Sign/Date) |
| --- | --- | --- | --- | --- | --- | --- |