# CLEANING VERIFICATION PROTOCOL

| Name of the product | : | # Isosorbide Mononitrate (ISM Stage-4) |
| --- | --- | --- |
| # Product Code | : | # ISM |
| Stage | : | # ISM4 |
| Plant | : | # Production Block-2 |

TABLE OF CONTENTS

1.0	APPROVAL SIGNATURES	3

2.0	OBJECTIVE	4

3.0	SCOPE	4

4.0	RESPONSIBILITIES	6

5.0	BACKGROUND AND CLEANING PROCEDURE	6

6.0	PRE-REQUISITES	8

7.0	EQUIPMENT QUALIFICATION STATUS	9

8.0	SURFACE AREA OF THE EQUIPMENT	11

9.0	RINSE VOLUME CALCULATION	12

10.0	MAXIMUM ALLOWABLE CARRYOVER (MACO)	14

11.0	ACCEPTANCE LIMIT CALCULATION (SWAB AND RINSE)	16

12.0	CLEANING VERIFICATION METHODOLOGY	19

13.0	SAMPLING PROCEDURE	21

14.0	DETERMINATION OF SWAB SAMPLE LOCATIONS:	24

15.0	SAMPLING PLAN, ACCEPTANCE CRITERIA AND CLEANING VALIDATION RESULTS SUMMARY:	25

15.1	MIXED VESSEL (MV-1304):	25

15.2	MIXED VESSEL (MV-1305):	32

15.3	PLUG FLOW REACTOR (PFR-1301):	39

15.4	GLASS LINED REACTOR (GLR-1305):	41

15.5	GLASS LINED REACTOR (GLR-1304):	48

15.6	GLASS LINED REACTOR (GLR-1303):	55

15.7	STAINLESS STEEL REACTOR (SSR-1305):	62

15.8	AGITATED NUTSCHE FILTER (ANF-1301):	69

15.9	LEAF FILTER (LF-1302):	76

15.10	AGITATED NUTSCHE FILTER CUM DRIER (ANFD-1301):	83

15.11	NITROSAMINE LIMITS IN THE RINSE SAMPLES:	90

15.12	POTENTIAL GENOTOXIC IMPURITIES LIMITS IN THE RINSE SAMPLES:	91

16.0	TESTING PROCEDURE	92

17.0	STATUS OF CLEANING ANALYTICAL METHOD VALIDATION	93

18.0	EVALUATION OF RESULTS AND REPORTING	94

19.0	DEVIATIONS	94

20.0	REVALIDATION	94

21.0	ABBREVIATIONS	95

22.0	RELATED DOCUMENTS	96

23.0	LIST OF ANNEXURES	96

24.0	HISTORY OF THE DOCUMENT	96

# APPROVAL SIGNATURES

| Function | Department | Name | Designation | Sign & date |
| --- | --- | --- | --- | --- |
| Prepared by | Quality Assurance |  |  |  |
| Reviewed by | Production |  |  |  |
|  | Technology Transfer |  |  |  |
|  | Quality Control |  |  |  |
|  | Engineering |  |  |  |
|  | Quality Assurance |  |  |  |
| Approved by | Quality Assurance |  |  |  |

# OBJECTIVE

Objective of this protocol is to conduct Cleaning verification of the manufacturing equipment used in the production of Isosorbide Mononitrate (Stage-4) to ensure the effective removal of residues, contaminants, and cleaning solvents to predefined acceptable levels. This verification will demonstrate that residual product from previous manufacturing cycles remains within acceptable limits when the equipment is cleaned using the defined cleaning method, thereby ensuring product quality and preventing cross-contamination.

This verification will assess the effectiveness of the cleaning procedures, the suitability of the facility (Production Block-2), equipment, materials, personnel, and established protocols to prevent cross-contamination and ensure compliance with regulatory and quality requirements. The execution of this validation is in alignment with Change Control reference# PCC-PRD-26-168.

The verification shall also demonstrate that small molecule nitrosamine impurities and potential genotoxic impurities are not formed or within their predefined acceptance limits after cleaning.

# SCOPE

This protocol applies to the cleaning verification activities for the manufacturing equipment given below used in the production of Isosorbide Mononitrate Stage-4 (ISM4) within Production Block-2 at 3xper Innoventure Limited, Naidupeta. It encompasses all relevant equipment surfaces that come into direct or indirect contact with the product during manufacturing operations.

| S. No. | Name of the Equipment | Equipment No. | Capacity | MOC | Purpose | Product contact / Non-product contact |
| --- | --- | --- | --- | --- | --- | --- |
|  | Mixing vessel | MV-1304 | 20 KL | SS316 | ISM3 holding | Product contact (ISM3) |
|  | Mixing vessel | MV-1305 | 12 KL | SS316 | Input for PFR reaction (ISM3) |  |
|  | Plug flow Reactor | PFR-1301 | 5 L | HAS | Reaction | Product contact (ISM4A) |
|  | Glass lined Reactor | GLR-1304 | 16 KL | MSGL | Quenching of reaction mass & extraction of ISM4A product | Product contact (ISM4A) |
|  | Glass lined Reactor | GLR-1305 | 10 KL | MSGL | Quenching of reaction mass & extraction of ISM4A product Initial MDC distillation from ISM3 | Product contact (ISM4A & ISM3) |
|  | Vessel | VES-1332 | 15 KL | CS halar | ISM4A reaction mass holding | Product contact (ISM4A) |
|  | ML Tank | MLT-1302 | 8 KL | SS 316 |  |  |
|  | Vessel | VES-1331 | 15 KL | SS 316 | Product filtrate collection from ANF | Product contact (ISM4) |
|  | Vessel | VES-1330 | 10 KL | SS 316 | Reaction mass charging into cyclohexane |  |
|  | Glass lined Reactor | GLR-1303 | 25 KL | MSGL | Reaction |  |
|  | Agitated Nutsche filter | ANF-1301 | 3 KL | SS316 | Salt Filtration |  |
|  | ML tank | MLT-1307 | 1 KL | SS 316 | Product filtrate collection |  |
|  | Vessel | VES-1331 | 15 KL | SS 316 | Product filtrate collection |  |
|  | Leaf filter | LF-1302 | 150 L | SS316 | Carbon filtration from product filtrate |  |
|  | Micron filter | MF-1302 | 20” | SS304 |  |  |
|  | Stainless steel Reactor | SSR-1305 | 10 KL | SS316L | Distillation & Isolation |  |
|  | Vessel | VES-1330 | 10 KL | SS 316 | Product holding before charging into cyclohexane |  |
|  | Agitated Nutsche filter drier | ANFD-1301 | 7 KL | SS316 | Product Filtration |  |
|  | ML tank | MLT-1301 | 0.5 KL | SS 316 | ML collection |  |
|  | Vessel | VES-1339 | 10 KL | MSGL | Holding of Fuming nitric acid (Input for PFR reaction) | Non product contact |
|  | Micron filter | MF-1303 | 20” | SS304 | Nitric acid filtration |  |
|  | Vessel | VES-1324 | 5 KL | SS 316 | Sodium methoxide solution charging |  |
|  | Vessel | VES-1335 | 3.5 KL | SS 316 | Hyflo slurry preparation |  |
|  | Mixing vessel | MV-1303 | 500 L | SS316 | Charcoal slurry preparation |  |

# RESPONSIBILITIES

| Department | Responsibility |
| --- | --- |
| Quality Assurance | - Preparation and review of the Cleaning Verification Protocol and Report. - Collection of swab samples from the equipment and verification of visual cleanliness. - Monitor the cleaning and sampling activities for compliance with the approved protocol and batch cleaning record. - Initiate and lead investigations for deviations, OOS results or non-conformances observed during execution. |
| Production | - Execution of the cleaning activity - Collection of rinse samples from the equipment. - Compile and document cleaning execution data following established SOPs. - Submission of the samples to QC for analysis with TRF. |
| Quality Control | - ﻿﻿Review and approval of Cleaning Verification Protocol and report. - Ensure that all cleaning analytical methods, are validated before sample analysis. - Analyze samples as per the approved Standard Testing Procedure (STP). - ﻿﻿Ensure timely and accurate reporting of analytical data. - Coordinate with production personnel for sample analysis and data review. - Support investigations for deviations, Out-of-Specification (OOS) results, or non-conformances observed during analysis. |
| Head / In-Charge (Technical Services) | - ﻿﻿Review and approval of Cleaning Verification Protocol and report. - ﻿﻿Monitor and assess process performance, reviewing results for consistency and compliance. |
| Engineering | - Review the Cleaning Verification Protocol and support cleaning execution. - Ensure that all equipment and utilities are operating within specified parameters and are qualified for use. - Equipment dismantling and re-fixing for cleaning activity, if required. |
| Head-Quality | - Approval of the Cleaning Verification Protocol and give clearance for further proceedings. - Approval of the Cleaning Verification Report and release of the equipment for further use. |

# BACKGROUND AND CLEANING PROCEDURE

Equipment chain will be cleaned after the manufacturing of Isosorbide Mononitrate (Stage-4) according to the defined procedure mentioned below. Cleaning Validation is the methodology used to evaluate and assure that a cleaning process removes residues of the Intermediate of the product manufactured in a piece of equipment / manufacturing equipment chain, the cleaning aids utilized in the cleaning process. All residues are removed to predetermined levels to ensure the quality of the next product manufactured is not compromised by waste from the previous product.

| S. No | Name of the Equipment | Equipment No. | Cleaning Method (BCR No.) | Cleaning Solvent |
| --- | --- | --- | --- | --- |
|  | Mixing vessel | MV-1304 |  | Potable Water and acetone |
|  | Glass lined Reactor | GLR-1305 |  | Potable Water and acetone |
|  | Mixing vessel | MV-1305 |  | Potable Water and acetone |
|  | Vessel | VES-1339 |  | Potable water and acetone |
|  | Plug flow Reactor | PFR-1301 |  | Potable Water and acetone |
|  | Glass lined Reactor | GLR-1304 |  | Potable Water and acetone |
|  | Glass lined Reactor | GLR-1305 |  | Potable Water and acetone |
|  | Mixing vessel | MV-1303 |  | Potable Water and acetone |
|  | Glass lined Reactor | GLR-1303 |  | Potable Water and acetone |
|  | Agitated Nutsche filter | ANF-1301 |  | Potable Water and acetone |
|  | Leaf filter | LF-1302 |  | Potable Water and acetone |
|  | Stainless steel Reactor | SSR-1305 |  | Potable Water and acetone |
|  | Agitated Nutsche filter drier | ANFD-1301 |  | Potable Water and acetone |
|  | ML tank | MLT-1301 |  | Potable Water and acetone |
| Note: The product is soluble in Water and Acetone. The detail cleaning process flow chart is given in Annexure-4. |  |  |  |  |

# PRE-REQUISITES

Prior to initiating cleaning verification, the following criteria, but not limited to, must be met to ensure process readiness and compliance with GMP requirements:

  - Critical Utilities & Support Systems:

The utilities and support systems (Nitrogen supply) must be assessed and demonstrated as adequate to support the intended manufacturing process.

  - Equipment Qualification:

All equipment of the train, associated auxiliary systems, critical utilities and analytical instruments must be qualified and within their valid qualification / calibration status as detailed in Section 7.0.

  - Standard Operating Procedures (SOPs):

Approved and effective SOPs for the operation and cleaning of equipment involved in the manufacturing process must be available and implemented.

  - Specifications & Analytical Methods:

Approved specifications and validated methods of analysis for cleaning solvents, final intermediate and cleaning residues must be available.

  - Batch Cleaning Record (BCRs) for cleaning must be prepared and approved before execution.

  - Raw Material Compliance:

    - All cleaning solvents must be sourced from approved vendors.

    - Cleaning solvents utilized in cleaning must comply with the respective approved specifications.

    - Acetone used for the final rinse shall be from a tested and released lot; the lot number shall be recorded in the BCR and a control sample of the same lot shall be retained and submitted to QC.

  - Sampling Plan: A well-defined sampling plan for the cleaning validation must be incorporated into the Batch Cleaning Record (BCR).

  - Calibration & Instrumentation:

    - All process-critical instruments, such as pressure gauges, temperature sensors, temperature indicators, and weighing balances, must be calibrated and within valid calibration status.

    - All monitoring and control instruments must be verified for accuracy before batch execution.

  - Training & Personnel Readiness: Manufacturing and quality control personnel involved in cleaning and sampling execution must receive documented training on the specific manufacturing process, equipment operation, sampling, and analytical procedures.

# EQUIPMENT QUALIFICATION STATUS

| Equipment No. | IQ protocol No & Effective date | IQ report No & Effective date | OQ protocol No & Effective date | OQ report No & Effective date | PQ protocol No & Effective date | PQ report No. & Effective date |
| --- | --- | --- | --- | --- | --- | --- |
| SSR-1305 | IQP/SSR-1305-00 | IQR/SSR-1305-00 | OQP/SSR-1305-00 | OQR/SSR-1305-00 | PQP/SSR-1305-00 | PQR/SSR-1305-00 |
|  | 14-07-2026 | 03-08-2026 | 14-08-2026 | 28-08-2026 | 28-08-2026 | 11-09-2026 |
| PFR-1301 | IQP/PFR-1301 | IQR/PFR-1301 | OQP/PFR-1301 | OQR/PFR-1301 | PQP/PFR-1301 | PQR/PFR-1301 |
|  | 02-09-2026 | 04-09-2026 | 16-09-2026 | 23-09-2026 | 24-09-2026 | U/progress |
| MV-1304 | IQP/MV-1304-00 | IQR/MV-1304-00 | OQP/MV-1304-00 | OQR/MV-1304-00 | NA | NA |
|  | 19-08-2026 | 25-08-2026 | 03-09-2026 | 11-09-2026 | NA | NA |
| MV-1305 | IQP/MV-1305-00 | IQR/MV-1305-00 | OQP/MV-1305-00 | OQR/MV-1305-00 | NA | NA |
|  | 14-08-2026 | 25-08-2026 | 03-09-2026 | 09-09-2026 | NA | NA |
| GLR-1304 | IQP/GLR-1304-00 | IQR/GLR-1304-00 | OQP/GLR-1304-00 | OQR/GLR-1304-00 | PQP/GLR-1304-00 | PQR/GLR-1304-00 |
|  | 14-07-2026 | 17-07-2026 | 14-08-2026 | 01-09-2026 | 27-08-2026 | 17-09-2026 |
| GLR-1305 | IQP/GLR-1305-00 | IQR/GLR-1305-00 | OQP/GLR-1305-00 | OQR/GLR-1305-00 | PQP/GLR-1305-00 | PQR/GLR-1305-00 |
|  | 14-07-2026 | 03-08-2026 | 12-08-2026 | 27-08-2026 | 27-08-2026 | 17-09-2026 |
| MV-1303 | IQP/MV-1303-00 | IQR/MV-1303-00 | OQP/MV-1303-00 | OQR/MV-1303-00 | NA | NA |
|  | 14-07-2026 | 17-07-2026 | 21-08-2026 | 09-09-2026 | NA | NA |
| GLR-1303 | IQP/GLR-1303-00 | IQR/GLR-1303-00 | OQP/GLR-1303-00 | OQR/GLR-1303-00 | PQP/GLR-1303-00 | PQR/GLR-1303-00 |
|  | 14-07-2026 | 17-07-2026 | 12-08-2026 | 29-08-2026 | 27-08-2026 | 18-09-2026 |
| ANF-1301 | IQP/ANF-1301-01 | IQR/ANF-1301-01 | OQP/ANF-1301-00 | OQR/ANF-1301-00 | PQP/ANF-1301-00 | PQR/ANF-1301-00 |
|  | 14-07-2026 | 20-07-2026 | 12-08-2026 | 27-08-2026 | 27-08-2026 | 11-09-2026 |
| LF-1302 | IQP/LF-1302-00 | IQR/LF-1302-00 | OQP/LF-1302-00 | OQR/LF-1302-00 | PQP/LF-1302-00 | PQR/LF-1302-00 |
|  | 14-07-2026 | 20-07-2026 | 21-08-2026 | 01-09-2026 | 28-08-2026 | 11-09-2026 |
| SSR-1305 | IQP/SSR-1305-00 | IQR/SSR-1305-00 | OQP/SSR-1305-00 | OQR/SSR-1305-00 | PQP/SSR-1305-00 | PQR/SSR-1305-00 |
|  | 14-07-2026 | 20-07-2026 | 14-08-2026 | 28-08-2026 | 28-08-2026 | 11-09-2026 |
| ANFD-1301 | IQP/ANFD-1301-00 | IQR/ANFD-1301-00 | OQP/ANFD-1301-00 | OQR/ANFD-1301-00 | PQP/ANFD-1301-00 | PQR/ANFD-1301-00 |
|  | 14-07-2026 | 10-08-2026 | 21-08-2026 | 01-09-2026 | 28-08-2026 | 18-09-2026 |

Inference: All equipment involved in ISM4 manufacturing has been qualified and is suitable for its intended use. Therefore, the equipment qualification requirements for this cleaning validation study are considered fulfilled.

# SURFACE AREA OF THE EQUIPMENT

  - Surface area values of the equipment and process lines have been calculated by Technology Transfer Team as mentioned below.

  - Calculation sheet for Surface area of process lines is attached as Annexure-1.

| S. No | Name of the Equipment | Equipment No. | Internal surface area (m²) |
| --- | --- | --- | --- |
|  | Glass Lined Reactor | GLR-1305 | 30.96 |
|  | Mixed Vessel | MV-1305 | 32.93 |
|  | Mixed Vessel | MV-1304 | 48.04 |
|  | Plug Flow Reactor | PFR-1301 | 0.23 |
|  | Glass Lined Reactor | GLR-1304 | 46.00 |
|  | Vessel | VES-1332 | 37.49 |
|  | ML Tank | MLT-1302 | 25.43 |
|  | Glass Lined Reactor | GLR-1303 | 60.66 |
|  | Agitated Nutsche Filter | ANF-1301 | 3.14 |
|  | ML Tank | MLT-1307 | 6.46 |
|  | Vessel | VES-1331 | 37.49 |
|  | Leaf filter | LF-1302 | 5.60 |
|  | Stainless Steel Reactor | SSR-1305 | 30.50 |
|  | Vessel | VES-1330 | 27.29 |
|  | Agitated Nutsche Filter and Drier | ANFD-1301 | 7.06 |
|  | ML Tank | MLT-1301 | 3.65 |
|  | Process lines surface area |  | 34.74 |
| Total surface area |  |  | 437.67 |

# RINSE VOLUME CALCULATION

The rinse volume for each equipment is established before the MACO and acceptance limit calculations, since the rinse acceptance limits (Section 11.0) are derived from the allowable carryover per equipment and the rinse volume given in this section. The same rinse volume shall be used during execution of the rinse sampling (Section 13.2).

  - Rinse solvent selection:

Acetone was selected as the final rinse solvent for the cleaning validation study based on its ability to effectively dissolve and recover residues of ISM3, ISM4A, ISM4, associated process-related impurities, and potential nitrosamine contaminants from product-contact surfaces. Acetone is fully miscible with water, facilitates rapid drying of equipment surfaces, and is compatible with the materials of construction of the process equipment. In addition, the target analytes exhibit adequate solubility and analytical recovery in acetone, making it a suitable solvent for the collection and analysis of rinse samples. Therefore, acetone rinse sampling was considered appropriate to demonstrate the effectiveness of the cleaning procedure and to assess residual contamination levels following completion of cleaning.

  - Calculation based on the Rinse Factor

    - Rinse Volume (L) = Surface Area (m²) × Rinse Factor (L/m²)

Where:

Surface Area (SA) = Internal surface area of the equipment/vessel

Rinse Factor = Typically ranges between 1–10 L/m²

1 - 3 L/m² → Easily soluble residues (e.g., APIs with solvent solubility)

3 - 5 L/m² → Moderately soluble residues

5 - 10 L/m² → Poorly soluble or potent residues (e.g., highly potent drugs, sticky residues)

    - The Isosorbide Mononitrate Stage-3, Stage-4A & Stage-4 are highly acetone soluble, hence Rinse Factor (L/m²) consider as 3 L/m² and calculate the rinse volume.

  - Calculation based on Solvent Adherence Factor (SAF)

    - Determine the Required Rinse Volume (Vₜ) = Va×SF

where:

Vₜ= Total rinse volume (L)

Va​ = Initial adhered water volume (L)

SF = Safety factor (Typically 6–10 for pharma industries)

    - Initial adhered solvent volume (L) Va​=A×SAF

where:

A = Surface area (m²)

SAF = Solvent adherence factor (L/m²)

    - The Solvent Adherence Factor (SAF) is a measure of how much water adheres to a surface after wetting and before it naturally drains or evaporates. It is expressed in liters per square meter (L/m²) and depends on surface properties, below are the surface types that have been used in this manufacturing chain

| S. No | Surface Type | WAF (L/m²) |
| --- | --- | --- |
| 1 | Polished Stainless Steel | 0.1-0.3 |
| 2 | Rough Stainless Steel | 0.3-0.6 |
| 3 | Glass Lined | 0.2-0.5 |
| 4 | PTFE / Halar surface | 0.1-0.3 |

  - Rinse Volume Calculation:

| S. No | Name of the Equipment | Equipment No. | Capacity | Internal surface area m² | Rinsing Volume (L) based on RF Formula= SA X RF |  | Rinsing Volume based on SAF Formula = SAXSAFXSF |  | Considered volume | Rinsing Sample Quantity |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Glass Lined Reactor | GLR-1305 | 10 KL | 30.96 | 30.96X 3 | 93 | 30.96 X 0.2 X6 | 37 | 37 L | 10 ml |
|  | Mixed Vessel | MV-1305 | 12 KL | 32.93 | 32.93X 3 | 99 | 32.93 X 0.1 X6 | 30 | 20 L | 10 ml |
|  | Mixed Vessel | MV-1304 | 20 KL | 48.04 | 48.04X 3 | 144 | 48.04 X 0.1 X6 | 29 | 29 L | 10 ml |
|  | Plug Flow Reactor | PFR-1301 | 5 L | 0.23 | 0.23 X 3 | 0.69 | 0.23 X 0.1 X 6 | 0.14 | 5 L | 10 ml |
|  | Glass Lined Reactor | GLR-1304 | 16 KL | 46.00 | 46 X 3 | 138 | 46.00 X 0.2 X6 | 55 | 55 L | 10 ml |
|  | Vessel | VES-1332 | 15 KL | 37.49 | 37.49X 3 | 112 | 37.49 X 0.1 X6 | 22 | 22 L | 10 ml |
|  | ML Tank | MLT-1302 | 8 KL | 25.43 | 25.43X 3 | 76 | 25.43 X 0.1 X6 | 15 | 15 L | 10 ml |
|  | Glass Lined Reactor | GLR-1303 | 25 KL | 60.66 | 60.66X3 | 182 | 60.66 X 0.2 X6 | 73 | 73 L | 10 ml |
|  | Agitated Nutsche Filter | ANF-1301 | 3 KL | 3.14 | 3.14X 3 | 9 | 3.14 X 0.1 X 6 | 2 | 2 L | 10 ml |
|  | ML Tank | MLT-1307 | 1 KL | 6.46 | 6.46X 3 | 19 | 6.46 X 0.1 X 6 | 4 | 4 L | 10 ml |
|  | Vessel | VES-1331 | 15 KL | 37.49 | 37.49X 3 | 112 | 37.49 X 0.1 X6 | 22 | 22 L | 10 ml |
|  | Leaf filter | LF-1302 | 150 L | 5.60 | 5.6 X 3 | 17 | 5.60 X 0.1 X 6 | 3 | 3 L | 10 ml |
|  | Stainless Steel Reactor | SSR-1305 | 10 KL | 30.50 | 30.5X 3 | 92 | 30.50 X 0.1 X6 | 18 | 18 L | 10 ml |
|  | Vessel | VES-1330 | 10 KL | 27.29 | 27.29X 3 | 82 | 27.29 X 0.1 X6 | 16 | 16 L | 10 ml |
|  | ANFD | ANFD-1301 | 7 KL | 7.06 | 7.06X 3 | 21 | 7.06 X 0.1 X 6 | 4 | 4 L | 10 ml |
|  | ML Tank | MLT-1301 | 0.5 KL | 3.65 | 3.65X 3 | 11 | 3.65 X 0.1 X 6 | 2 | 2 L | 10 ml |

Note: Based on the above calculation, the rinsing volume shall be determined using the SAF, as it represents the worst-case scenario (lowest rinse volume, i.e. highest residue concentration).

# MAXIMUM ALLOWABLE CARRYOVER (MACO)

Maximum Allowable Carryover (MACO) ensures that the quality and safety of subsequent products are not compromised by residual contaminants from previous batches.

| S. No. | Name of the Equipment | Equipment No. | Capacity | MOC | Is used for Stage-4? | Equipment used previous / subsequent to this product | Minimum batch size for this equipment manufactured | Product contact / Non-product contact |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Mixed Vessel | MV-1305 | 12 KL | SS 316L | Yes | No | 771 Kg | Product contact (ISM3) |
| 2 | Mixed Vessel | MV-1304 | 20 KL | SS 316L | Yes | No | 771 Kg |  |
| 3 | Plug Flow Reactor | PFR-1301 | 5 L | SS316 | Yes | No | 771 Kg | Product contact (ISM4A) |
| 4 | Glass Lined Reactor | GLR-1304 | 16 KL | MSGL | Yes | No | 771 Kg |  |
| 5 | Glass Lined Reactor | GLR-1305 | 10 KL | MSGL | Yes | No | 771 Kg |  |
|  | Vessel | VES-1332 | 15 KL | CS Halar | Yes | No | 771 Kg |  |
| 6 | ML Tank | MLT-1302 | 8 KL | SS 316 | Yes | No | 771 Kg |  |
| 7 | Glass Lined Reactor | GLR-1303 | 25 KL | MSGL | Yes | No | 771 Kg | Product contact (ISM4) |
| 8 | Agitated Nutsche Filter | ANF-1301 | 3 KL | SS 316 | Yes | No | 771 Kg |  |
| 9 | ML Tank | MLT-1307 | 1 KL | SS 316 | Yes | No | 771 Kg |  |
| 10 | Vessel | VES-1331 | 15 KL | SS 316 | Yes | No | 771 Kg |  |
| 11 | Leaf filter | LF-1302 | 150 L | SS 316L | Yes | No | 771 Kg |  |
| 12 | Stainless Steel Reactor | SSR-1305 | 10 KL | SS 316L | Yes | No | 771 Kg |  |
| 13 | Vessel | VES-1330 | 10 KL | SS 316 | Yes | No | 771 Kg |  |
| 14 | ANFD | ANFD-1301 | 7 KL | SS 316 | Yes | No | 771 Kg |  |
| 15 | ML Tank | MLT-1301 | 0.5 KL | SS 316 | Yes | No | 771 Kg |  |
| 16 | Vessel | VES-1339 | 10 KL | MSGL | Yes | No | - | Non product contact |
| 17 | Micron filter | MF-1303 | 20” | SS304 | Yes | No | - |  |
| 18 | Vessel | VES-1324 | 5 KL | SS 316 | Yes | No | - |  |
| 19 | Vessel | VES-1335 | 3.5 KL | SS 316 | Yes | No | - |  |
| 20 | Mixing vessel | MV-1303 | 500 L | SS316 | Yes | No | - |  |
| Note: A batch size of 250 kg has been considered as the worst-case minimum batch size for MACO calculation, representing one-third of the current minimum commercial batch size of 771 kg, to provide a conservative assessment for potential future manufacturing campaigns with lower batch sizes in PB-2. Non-product equipment shall be tested for pH and extraneous matter. |  |  |  |  |  |  |  |  |

*Existing product / stage (i.e. ISM Stage-4) is considered as subsequent product / stage as these equipment are not yet used for any other product / stage.

Maximum Allowable Carryover (MACO) calculation using health-based data:

| Attribute | Description of Attribute | Value |
| --- | --- | --- |
| PDE | PDE value of the ISM | #0.12 mg/day |
| MBS | The minimum batch size for the subjected equipment | 250 Kg = 250000000 mg |
| TDD | Therapeutic daily dose of subsequent product | ##4000 mg |
| MACO | PDE X MBS --------------- TDD | 0.12 X 250000000 --------------------- 4000 = 7500 mg |
| # PDE report is attached as Annexure-2. ## Reference guideline for TDD is attached as Annexure-3. |  |  |

Maximum Allowable Carryover (MACO) calculation using general limit:

| Attribute | Description of Attribute | Calculation / MACO value |
| --- | --- | --- |
| MAXCONC | The allowable limit to carryover for intermediate per equipment train of product | *10 ppm 10 mg Kg |
| MBS | The minimum batch size considered for Production Block-2 | 250 Kg |
| MACO | MBS X MAXCONC | 250 Kg X 10 mg/Kg = 2500 mg |
| The 10 ppm residue limit has been considered as the worst-case acceptance criterion for cleaning validation, in preference to the generally acceptable 100 ppm limit for intermediate manufacturing facilities, to provide a conservative margin of safety. |  |  |

Selection of MACO: The health-based MACO (7500 mg) and the general limit MACO (2500 mg) have been compared, and the lower value, i.e. 2500 mg, is considered for the calculation of swab and rinse acceptance limits for product residue (ISM4 content).

# ACCEPTANCE LIMIT CALCULATION (SWAB AND RINSE)

  - Total surface area of equipment train mapped for the product in Production block-2 is 437.67 m2.

  - The allowable carryover per m2 = MACO ÷ Total surface area = 2500 mg ÷ 437.67 m2 ≈ 6 mg/m2.

  - The minimum batch size of the subsequent product in Production Block-2 is 250 Kg.

  - The allowable carryover per equipment shall be calculated as given below.

  - Swab Limit:

| Attribute | Description of Attribute | Value / Calculation |
| --- | --- | --- |
| Equipment Surface Area | Total surface area of shared equipment train for this product (Refer Section 8.0). | 437.67 m² |
| Acceptance Limit per m² | Maximum allowable residue on equipment surfaces after cleaning, expressed per square meter. Calculation: MACO ÷ Total Surface Area | 2500 mg ÷ 437.67 m² = 6 mg/m² |
| Swab size | Actual size of the absorbent part of the swab | 0.01 m2 or 100 cm2 |
| Swab Limits | The allowable residue on the swab after cleaning. Calculation: Acceptance Limit per m² × Surface area of swab (m²) | 6 mg/m² × 0.01 m² = 0.06 mg |
| Diluent Volume (ml) | Measured quantity of solvent used to Prepare sample solutions for analysis in analysis | 10 ml |
| Acceptance Limit per Equipment (ppm) | The maximum allowable residue concentration on equipment surfaces after cleaning. Calculation: Acceptance Limit per Swab × 1000 ÷ Diluent Volume (ml) | 0.06 mg × 1000 ÷ 10 = 6 ppm |

  - Rinse Limit:

| Attribute | Description | Value / Calculation |
| --- | --- | --- |
| Equipment Surface Area | Total surface area of the shared equipment train used for this product (Refer Section 9.0). | 437.67 m² |
| Surface Acceptance Limit per m² | Maximum allowable residue on equipment surfaces after cleaning, expressed per square meter. Calculation: MACO ÷ Total Surface Area | 2500 mg ÷ 437.67 m² = 6 mg/m² |

| Equipment ID | Surface area of each equipment (in m2) | Allowable carryover per equipment (in mg) | Rinse volume (in L) | Rinse acceptance criteria (in ppm) | Final rinse acceptance criteria (in ppm) |
| --- | --- | --- | --- | --- | --- |
|  | B | C = A X B | D | E = C / D |  |
| GLR-1305 | 30.96 | 546 | 37 | 15 | 10 |
| MV-1305 | 32.93 | 580 | 20 | 29 | 10 |
| MV-1304 | 48.04 | 847 | 29 | 29 | 10 |
| PFR-1301 | 0.23 | 1.3 | 5 | 0.3 | 0.3 |
| GLR-1304 | 46.00 | 811 | 55 | 15 | 10 |
| VES-1332 | 37.49 | 661 | 22 | 29 | 10 |
| MLT-1302 | 25.43 | 448 | 15 | 29 | 10 |
| GLR-1303 | 60.66 | 1069 | 73 | 15 | 10 |
| ANF-1301 | 3.14 | 55 | 2 | 29 | 10 |
| MLT-1307 | 6.46 | 114 | 4 | 29 | 10 |
| VES-1331 | 37.49 | 661 | 22 | 29 | 10 |
| LF-1302 | 5.60 | 99 | 3 | 29 | 10 |
| SSR-1305 | 30.50 | 537 | 18 | 29 | 10 |
| VES-1330 | 27.29 | 481 | 16 | 29 | 10 |
| ANFD-1301 | 7.06 | 124 | 4 | 29 | 10 |
| MLT-1301 | 3.65 | 64 | 2 | 29 | 10 |

Note: *Final rinse acceptance criterion is restricted to NMT 10 ppm for all equipment as a worst case. Rinse volumes (D) are as calculated in Section 9.3.

# CLEANING VERIFICATION METHODOLOGY

  - Cleaning verification activities shall be performed in following sequence for each cleaning run:

    - Clean the equipment as per the approved Batch Cleaning Record.

    - Perform visual inspection. If the equipment is not visually clean, sampling shall not be performed; the equipment shall be re-cleaned and the event recorded as a deviation.

    - Visual Inspection: After the cleaning procedure is performed, the equipment shall be dried to allow the visual inspection. No residue shall be visible. Visual inspection shall be performed by Production and verified by QA using adequate illumination (e.g., torch light) wherever direct view is not possible. During visual inspection the following situations shall be considered:

      - Discolored surfaces, worn or torn parts;

      - Solid residues (for final product equipment used downstream of last filtration, residues should be evaluated also by passing final washing through a rough filter media (e.g., a lint-free cloth))

    - Collect the swab samples from the defined locations after the final rinse (refer Section 15.0).

    - Collect the final rinse samples (refer Section 15.0) and the acetone control sample.

    - Label the samples, record the sampling details and submit the samples to QC with RCTR.

    - Analyse the samples as per the testing procedures given in Sections 16.0 & 17.0

    - Evaluate the results against the acceptance criteria given in Section 15.0.

  - A visual test and a chemical evaluation of the equipment shall be performed after its cleaning to demonstrate that product residues have been removed to levels within the acceptance criteria.

  - The equipment cleanliness shall be demonstrated by visual inspection and by testing and evaluation of rinse and swab samples, in accordance with this protocol. Extraneous matter test shall be performed for rinse samples collected from all the equipment.

  - Small molecule nitrosamine impurities and potential genotoxic impurities testing shall be done for rinse samples of equipment MV-1304, GLR-1305, MV-1305, PFR-1301, GLR-1304, GLR-1303, SSR-1305 & ANFD-1201.

  - A report shall be prepared assessing the data generated, thereby determining the validity of the cleaning process (refer Section 18.0).

  - The equipment shall not be used to process another product until clearance, indicating that the equipment is adequately clean, has been received from the cleaning verification report in accordance with the procedure.

# SAMPLING PROCEDURE

This section defines methodologies for both swab and rinse sampling.

  - Swab sampling procedure

Swab sampling procedure using swab cloth:

    - Use a new swab cloth (4 x 4 inches) for swab sampling. Wear fresh hand gloves and take the cloth. Do not touch cloth directly to prevent contamination.

    - Immerse the swab cloth into the sample bottle containing 25 ml of cleaning verification solvent.

    - Gently remove the cloth from the sample bottle and squeeze it to remove any excess solvent, ensuring that no drips remain.

    - Using the cloth, wipe the designated sample location surface with horizontal strokes (moving from left to right) and vertical strokes (from up to down). Ensure that each stroke overlaps with the previous one, as shown in below Figure, to achieve full coverage.

HORIZONTAL STROKES FROM LEFT TO RIGHT

VERTICAL STROKES FROM UP TO DOWN

    - After completing the surface wiping, place the used cloth back into the sample bottle for analysis.

Swab sampling procedure using swab stick:

    - Take 10 ml of diluent solution in sample bottle to collect the swab sample.

    - Diluent used for swab sampling shall be the cleaning agent in final rinse.

    - Use a swab stick to collect the sample form the equipment surface in the area of 10 cm X 10 cm.

    - Wet the swab stick with diluent and swab a portion of sampling area vertically in top to bottom, likewise swab the same portion horizontally from left to right with other side of the stick and drop the swab head into the diluent solution and close the bottle.

  - Rinse sampling procedure

    - Ensure the Reactor / Receiver / ANFD / Centrifuge is free from visible residues and foreign particles.

    - Confirm that all valves, and drain points are functional.

    - Flush the Reactor / Receiver / ANFD / Centrifuge with sufficient water / solvent as mentioned in the Batch Cleaning Records (refer Section 5.0).

    - Drain completely through the bottom outlet.

    - Ensure the entire internal surface is covered in flushing, including hard-to-reach areas.

    - Allow water / solvent to flow through all ports, manways, and sampling points.

    - Make sure the rinsing solvent should reach all the portions of equipment.

    - Collect the rinse samples in clean, dry and labelled sample bottles and send them to the QC department for analysis along with the RCTR.

    - Collect a control sample (approx. 50 ml) of the unused acetone from the same lot used for the final rinse, in the same manner and at the same time. It can be used for blank testing during analysis of cleaning samples.

# DETERMINATION OF SWAB SAMPLE LOCATIONS:

Swab sampling shall be performed irrespective of equipment capacity or size. Product-contact surfaces that are difficult to clean, difficult to access, prone to residue accumulation, or represent worst-case cleaning locations shall be considered for swab sampling.

For reactors or other product-contact vessels, the top dish, bottom dish, manhole seating area, agitator assembly, discharge valve, and other identified worst-case product-contact surfaces shall be included as mandatory swab sampling locations. These locations represent areas prone to residue accumulation, difficult cleaning, or limited accessibility.

In addition to the above worst-case locations, additional swab samples shall be collected on shell wall surface. For vessel shell wall sampling, the number of horizontal sampling levels shall be established based on the shell height as follows:

- For equipment having a shell height of ≤ 2 m, a minimum of one horizontal sampling level shall be considered i.e. middle level.

- For equipment having a shell height of > 2 m, the number of horizontal sampling levels shall be determined using the following formula:

n= √H+1

Where:

- n = Number of horizontal sampling levels (rounded up to the next whole number)

- H = Equipment shell height (m)

The additional "+1" ensures adequate coverage of the shell surface and provides at least one additional sampling level beyond the square root calculation.

For vessels with a diameter ≤ 1 m, sampling shall be performed at two circumferential locations (0° and 180°) at each level.

For vessels having a diameter of > 1 m, sampling shall be performed at four circumferential locations (0°, 90°, 180°, and 270°) at each level.

# SAMPLING PLAN, ACCEPTANCE CRITERIA AND CLEANING VALIDATION RESULTS SUMMARY:

# MIXED VESSEL (MV-1304):

The subject equipment is located in Production Block-2, a multipurpose manufacturing facility. The equipment is qualified for its intended use, and the cleaning validation approach, including sampling locations, has been established based on the approved FMEA (Ref. No. FMEA/MV-1304-00). This equipment is used in ISM4 manufacturing for ISM3 intermediate material holding prior to charging batch.

    - Equipment details

The equipment details, including Material of Construction (MOC), product contact surface area, shell height, and shell diameter, were obtained from CPDR Annexure-2 and the applicable equipment qualification documents. These parameters were used to establish the shell wall swab sampling locations.

| Parameter | Details | Reference |
| --- | --- | --- |
| Capacity | 20 KL | CPDR Annexure-2 |
| MOC | SS 316L | CPDR Annexure-2 |
| Surface Area | 48.04 m2 | CPDR Annexure-2 |
| Shell height | 3908 mm | CPDR Annexure-2 |
| Shell diameter | 2800 mm | CPDR Annexure-2 |
| Type of agitator | Anchor | IQ report#: IQR/MV-1304-00 |
| No. of baffles | 4 | IQ report#: IQR/MV-1304-00 |

    - Supporting Documents and References

| Documents | Document # | Effective / Approval date |
| --- | --- | --- |
| BCR | PB2-BCR-ISM4-XXX-00 |  |
| Specification | QCD-CLS-ISM3-001-00 | 15-09-2025 |
| Testing Procedure | QCD-CLT-ISMC-001-00 | 15-09-2025 |
| Analytical Method Validation | IMN-VAL-FP-CLM-25-015 | 15-10-2025 |
| Equipment Qualification (OQ) | OQR/MV-1304-00 | 11-09-2026 |

    - Swab sampling locations determination:

      - Worst-case locations:

The following worst-case product-contact locations shall be included as mandatory swab sampling locations based on equipment design, accessibility, cleanability, and residue accumulation potential.

| Location ID | Description of location |
| --- | --- |
| S-1 | Vessel top dish near manhole |
| S-2 | Vessel bottom side inner surface |
| S-3 | Manhole lid seating area |
| S-4 | Vessel’s Stirrer |
| S-5 | Vessel’s stirrer blade |
| S-6 | Bottom discharge valve |

      - Calculation for shell wall swab locations:

The number of shell wall swab sampling locations has been determined from the vessel shell height and diameter, to ensure representative coverage of the internal product-contact surface. Refer to Section 14.0.

| Parameter | Calculation | Value | Remarks |
| --- | --- | --- | --- |
| Shell height (H) | NA | 3.9 m | Measured from the bottom tangent line to the top tangent line |
| No. of horizontal levels (n = √H+1) | √3.9+1 | 3 #s | Rounded to the nearest whole number. Levels at 0 m, 2 m and 3.9 m height |
| Shell Diameter (D) | NA | 2.8 m | D > 1 m, hence the four-location criterion applies |
| No. of circumferential locations (C) Case 1: D<1 = two locations Case 2: D>1 = four locations | NA | 4 #s | Four circumferential locations (0°, 90°, 180° and 270°) at each horizontal level, as D = 2.8 m falls under Case 2 |
| Total No. of samples on wall surface (n × C) | 4 x 3 | 12 #s | Total of 12 swab locations on the shell wall surface |

      - Pictorial representation of MV-1304:

The following are the pictorial representation of the swab sampling locations as per identified worst-case product-contact locations;

S-1

S-3

S-2

S-7a to S-7d

(0 meter level)

S-4

S-5

S-6

S-8a to S-8d

(2 meter level)

S-9a to S-9d

(3.9 meter level)

      - Rationale for swab sample locations:

Swab sampling locations have been selected on a risk basis from the equipment design and operation, considering accessibility, cleanability, residue accumulation potential and the extent of product-contact surface. They comprise the mandatory worst-case locations (S-1 to S-6) and representative locations on the vessel shell wall (S-7 to S-9), the number of which is derived in Section 14.0.

| Swab ID. # | Description | ###### Rationale (Based on operation and design of equipment) | ###### No. of samples |
| --- | --- | --- | --- |
| S-1 | Vessel top dish near manhole | Product contact surface Hard to clean (overhead surface, shadowed from direct spray) Potential for splash and residue deposition | 1 |
| S-2 | Vessel bottom side inner surface near discharge | Product contact surface Product accumulation (lowest point) Hard to reach Likely worst-case for residue retention | 1 |
| S-3 | Manhole lid seating area | Hard to clean (gasket and crevice) Seam and joint geometry Potential for residue entrapment | 1 |
| S-4 | Stirrer shaft, middle location | Product contact Hard to reach Irregular shape | 1 |
| S-5 | Stirrer blade | Product contact surface Hard to reach Irregular shape (blade and hub junction) Potential spray shadow | 1 |
| S-6 | Bottom discharge valve | Product contact surface Hard to clean Seam and gasket Irregular shape Lowest point, flow dead-leg potential | 1 |
| S-7a, S-7b, S-7c, S-7d | Vessel shell wall at 0 m level (0°, 90°, 180°, 270°) | Product contact surface Representative of the largest contact area Coverage of the lower shell, where product accumulates | 4 |
| S-8a, S-8b, S-8c, S-8d | Vessel shell wall at 2 m level (0°, 90°, 180°, 270°) | Product contact surface Representative of the largest contact area Coverage of the mid shell | 4 |
| S-9a, S-9b, S-9c, S-9d | Vessel shell wall at 3.9 m level (0°, 90°, 180°, 270°) | Product contact surface Representative of the largest contact area Coverage of the upper shell, where splash and vapor condensation can deposit residue | 4 |

    - Cleaning operation results summary:

The cleaning operation was performed in accordance with the approved cleaning procedure and associated cleaning instructions. Critical cleaning process parameters, including cleaning solvent quantity, heating temperature, heating time, cooling temperature, cooling time, and overall cleaning cycle duration, shall be recorded for each cleaning verification batch. The recorded process parameters shall be reviewed to confirm compliance with the approved cleaning procedure and to demonstrate consistency and reproducibility of the cleaning process throughout the cleaning verification study.

The results of the cleaning operation for each equipment and associated process line are summarized in the following table. Any deviation from the approved cleaning parameters shall be documented, investigated, and assessed for potential impact on the effectiveness of cleaning and the validity of the cleaning verification results.

The completed cleaning operation shall be considered satisfactory only when the recorded cleaning parameters are within the approved operating ranges and subsequent visual inspection, manufacturing area verification, process line verification, and analytical testing results comply with the established acceptance criteria.​‌

| Cleaning Parameter | Acceptance Criteria / Target | ISM1 Batch No. ______ | ISM2 Batch No. ______ | ISM3 Batch No. ______ |
| --- | --- | --- | --- | --- |
| Water Quantity Used for flushing (L) | As per approved cleaning SOP |  |  |  |
| Observed level of adhered material | Minimal |  |  |  |
| Water quantity used for reflux |  |  |  |  |
| Water Heating Temperature (°C) |  |  |  |  |
| Water Heating Time (Min) |  |  |  |  |
| Water Contact / Recirculation Time (Min) |  |  |  |  |
| Water Cooling Temperature (°C) |  |  |  |  |
| Water Cooling Time (Min) |  |  |  |  |
| Adhered material level | Minimal |  |  |  |
| Rinse sample test result after water reflux |  |  |  |  |
| Water cleaning cycle time |  |  |  |  |
| Acetone Quantity Used (L) | As per approved cleaning SOP |  |  |  |
| Acetone Heating Temperature (°C) (if applicable) |  |  |  |  |
| Acetone Contact / Recirculation Time (Min) |  |  |  |  |
| Acetone Cooling Temperature (°C) |  |  |  |  |
| Acetone Cooling Time (Min) |  |  |  |  |
| Acetone cleaning cycle time |  |  |  |  |
| Total Cleaning Cycle Time (Min) |  |  |  |  |
| Rinse volume of acetone |  |  |  |  |
| Rinse operation time |  |  |  |  |
| Rinse test result |  |  |  |  |

Inference:

Conclusion:

    - Cleaning validation results summary:

Upon completion of cleaning, the equipment shall undergo visual inspection under qualified inspection conditions. Following satisfactory visual inspection, swab samples shall be collected from predefined worst-case locations and final rinse samples shall be collected from the equipment. The results of visual inspection and analysis of swab, and rinse samples shall be evaluated against the established acceptance criteria to demonstrate the effectiveness and consistency of the cleaning procedure.

    - Visual inspection summary

As a primary verification of equipment cleanliness, visual inspection shall be conducted under established inspection conditions to assess the presence of visible product residues, foreign matter, staining, or other indications of inadequate cleaning. The observations made by the Production Chemist, Production Shift In-charge, and QA representative during each cleaning validation run are presented in the following table. Any permanent stains observed shall be evaluated to confirm that they do not impact the effectiveness of the cleaning process or product quality.

| Sample description / location | Results |
| --- | --- |
|  | B. No. 3ISMPP4261001 |
| Production Chemist verification |  |
| Production Shift in-charge verification |  |
| QA verification |  |
| Is any permanent stain recorded? |  |

    - Reflux, Swab & Rinse samples analysis results summary (ISM3 content):

The following table summarizes the sampling locations and results for ISM3 residue analysis during the cleaning validation study. Results from all validation runs shall be compared against the established acceptance criterion of NMT 10 ppm.

| Sample description / location | Sample ID | Results (ISM3 content) |
| --- | --- | --- |
|  |  | B. No. 3ISMPP4261001 |
| Final Rinse Sample (Acetone) | NA |  |
| Swab samples |  |  |
| Vessel top dish near manhole | S-1 |  |
| Vessel bottom side inner surface near discharge | S-2 |  |
| Manhole lid seating area | S-3 |  |
| Stirrer shaft, middle location | S-4 |  |
| Stirrer blade | S-5 |  |
| Bottom discharge valve | S-6 |  |
| Vessel shell wall at 0 m level (0°) | S-7a |  |
| Vessel shell wall at 0 m level (90°) | S-7b |  |
| Vessel shell wall at 0 m level (180°) | S-7c |  |
| Vessel shell wall at 0 m level (270°) | S-7d |  |
| Vessel shell wall at 2 m level (0°) | S-8a |  |
| Vessel shell wall at 2 m level (90°) | S-8b |  |
| Vessel shell wall at 2 m level (180°) | S-8c |  |
| Vessel shell wall at 2 m level (270°) | S-8d |  |
| Vessel shell wall at 3.9 m level (0°) | S-9a |  |
| Vessel shell wall at 3.9 m level (90°) | S-9b |  |
| Vessel shell wall at 3.9 m level (180°) | S-9c |  |
| Vessel shell wall at 3.9 m level (270°) | S-9d |  |
| Limit |  | NMT 10 ppm |
| LOQ |  | NMT 5 ppm |
| LOD |  | NMT 2 ppm |

    - Rinse samples analysis results summary (Extraneous matter):

As part of the cleaning validation study, final rinse samples shall be evaluated for extraneous matter to confirm that the cleaning process effectively removes visible foreign contaminants from product-contact surfaces. The examination shall include assessment for black particles, fibers, and other extraneous matter. The results from all validation runs are presented in the following table and shall be evaluated against the acceptance criterion that no black or fiber particles are observed in the rinse samples.

| Sample description / location | Sample ID | Results (Extraneous matter) |
| --- | --- | --- |
|  |  | B. No. 3ISMPP4261001 |
| Rinse Sample | NA |  |
| Limit |  | Black and fiber particles should be absent |

Inference:

It shall be written in the cleaning verification report.

Conclusion:

It shall be written in the cleaning verification report.

# MIXED VESSEL (MV-1305):

The subject equipment is located in Production Block-2, a multipurpose manufacturing facility. The equipment is qualified for its intended use, and the cleaning validation approach, including sampling locations, has been established based on the approved FMEA (Ref. No. FMEA/MV-1305-00). This equipment is used in ISM4 manufacturing for ISM3 intermediate material for plug flow reactor input (after initial MDC distillation).

    - Equipment details

The equipment details, including Material of Construction (MOC), product contact surface area, shell height, and shell diameter, were obtained from CPDR Annexure-2 and the applicable equipment qualification documents. These parameters were used to establish the shell wall swab sampling locations.

| Parameter | Details | Reference |
| --- | --- | --- |
| Capacity | 12 KL | CPDR Annexure-2 |
| MOC | SS 316L | CPDR Annexure-2 |
| Surface Area | 32.93 m2 | CPDR Annexure-2 |
| Shell height | 3540 mm | CPDR Annexure-2 |
| Shell diameter | 2200 mm | CPDR Annexure-2 |
| Type of agitator | Anchor | IQ report#: IQR/MV-1305-00 |
| No. of baffles | 4 | IQ report#: IQR/MV-1305-00 |

    - Supporting Documents and References

| Documents | Document # | Effective / Approval date |
| --- | --- | --- |
| BCR | PB2-BCR-ISM4-XXX-00 |  |
| Specification | QCD-CLS-ISM3-001-00 | 15-09-2025 |
| Testing Procedure | QCD-CLT-ISMC-001-00 | 15-09-2025 |
| Analytical Method Validation | IMN-VAL-FP-CLM-25-015 | 15-10-2025 |
| Equipment Qualification (OQ) | OQR/MV-1305-00 | 09-09-2026 |

    - Swab sampling locations determination:

      - Worst-case locations:

The following worst-case product-contact locations shall be included as mandatory swab sampling locations based on equipment design, accessibility, cleanability, and residue accumulation potential.

| S-1 | Vessel top dish near manhole |
| --- | --- |
| S-2 | Vessel bottom side inner surface |
| S-3 | Manhole lid seating area |
| S-4 | Vessel’s Stirrer |
| S-5 | Vessel’s stirrer blade |
| S-6 | Bottom discharge valve |

      - Calculation for shell wall swab locations:

The number of shell wall swab sampling locations has been determined from the vessel shell height and diameter, to ensure representative coverage of the internal product-contact surface. Refer to Section 14.0.

| Parameter | Calculation | Value | Remarks |
| --- | --- | --- | --- |
| Shell height (H) | NA | 3.5 m | Measured from the bottom tangent line to the top tangent line |
| No. of horizontal levels (n = √H+1) | √3.5+1 | 3 #s | Rounded to the nearest whole number. Levels at 0 m, 2 m and 3.5 m height |
| Shell Diameter (D) | NA | 2.2 m | D > 1 m, hence the four-location criterion applies |
| No. of circumferential locations (C) Case 1: D<1 = two locations Case 2: D>1 = four locations | NA | 4 #s | Four circumferential locations (0°, 90°, 180° and 270°) at each horizontal level, as D = 2.2 m falls under Case 2 |
| Total No. of samples on wall surface (n × C) | 4 x 3 | 12 #s | Total of 12 swab locations on the shell wall surface |

      - Pictorial representation of MV-1305:

The following are the pictorial representation of the swab sampling locations as per identified worst-case product-contact locations;

S-1

S-3

S-2

S-7a to S-7d

(0 meter level)

S-4

S-5

S-6

S-8a to S-8d

(2 meter level)

S-9a to S-9d

(3.5 meter level)

      - Rationale for swab sample locations:

Swab sampling locations have been selected on a risk basis from the equipment design and operation, considering accessibility, cleanability, residue accumulation potential and the extent of product-contact surface. They comprise the mandatory worst-case locations (S-1 to S-6) and representative locations on the vessel shell wall (S-7 to S-9), the number of which is derived in Section 14.0.

| Swab ID. # | Description | ###### Rationale (Based on operation and design of equipment) | ###### No. of samples |
| --- | --- | --- | --- |
| S-1 | Vessel top dish near manhole | Product contact surface Hard to clean (overhead surface, shadowed from direct spray) Potential for splash and residue deposition | 1 |
| S-2 | Vessel bottom side inner surface near discharge | Product contact surface Product accumulation (lowest point) Hard to reach Likely worst-case for residue retention | 1 |
| S-3 | Manhole lid seating area | Hard to clean (gasket and crevice) Seam and joint geometry Potential for residue entrapment | 1 |
| S-4 | Stirrer shaft, middle location | Product contact Hard to reach Irregular shape | 1 |
| S-5 | Stirrer blade | Product contact surface Hard to reach Irregular shape (blade and hub junction) Potential spray shadow | 1 |
| S-6 | Bottom discharge valve | Product contact surface Hard to clean Seam and gasket Irregular shape Lowest point, flow dead-leg potential | 1 |
| S-7a, S-7b, S-7c, S-7d | Vessel shell wall at 0 m level (0°, 90°, 180°, 270°) | Product contact surface Representative of the largest contact area Coverage of the lower shell, where product accumulates | 4 |
| S-8a, S-8b, S-8c, S-8d | Vessel shell wall at 2 m level (0°, 90°, 180°, 270°) | Product contact surface Representative of the largest contact area Coverage of the mid shell | 4 |
| S-9a, S-9b, S-9c, S-9d | Vessel shell wall at 3.5 m level (0°, 90°, 180°, 270°) | Product contact surface Representative of the largest contact area Coverage of the upper shell, where splash and vapor condensation can deposit residue | 4 |

    - Cleaning validation results summary:

Upon completion of cleaning, the equipment shall undergo visual inspection under qualified inspection conditions. Following satisfactory visual inspection, swab samples shall be collected from predefined worst-case locations and final rinse samples shall be collected from the equipment. The results of visual inspection and analysis of swab, and rinse samples shall be evaluated against the established acceptance criteria to demonstrate the effectiveness and consistency of the cleaning procedure.

    - Visual inspection summary

As a primary verification of equipment cleanliness, visual inspection shall be conducted under established inspection conditions to assess the presence of visible product residues, foreign matter, staining, or other indications of inadequate cleaning. The observations made by the Production Chemist, Production Shift In-charge, and QA representative during each cleaning validation run are presented in the following table. Any permanent stains observed shall be evaluated to confirm that they do not impact the effectiveness of the cleaning process or product quality.

| Sample description / location | Results |
| --- | --- |
|  | B. No. 3ISMPP4261001 |
| Production Chemist verification |  |
| Production Shift in-charge verification |  |
| QA verification |  |
| Is any permanent stain recorded? |  |

    - Reflux, Swab & Rinse samples analysis results summary (ISM3 content):

The following table summarizes the sampling locations and results for ISM3 residue analysis during the cleaning validation study. Results from all validation runs shall be compared against the established acceptance criterion of NMT 10 ppm.

| Sample description / location | Sample ID | Results (ISM3 content) |
| --- | --- | --- |
|  |  | B. No. 3ISMPP4261001 |
| Final Rinse Sample (Acetone) | NA |  |
| Swab samples |  |  |
| Vessel top dish near manhole | S-1 |  |
| Vessel bottom side inner surface near discharge | S-2 |  |
| Manhole lid seating area | S-3 |  |
| Stirrer shaft, middle location | S-4 |  |
| Stirrer blade | S-5 |  |
| Bottom discharge valve | S-6 |  |
| Vessel shell wall at 0 m level (0°) | S-7a |  |
| Vessel shell wall at 0 m level (90°) | S-7b |  |
| Vessel shell wall at 0 m level (180°) | S-7c |  |
| Vessel shell wall at 0 m level (270°) | S-7d |  |
| Vessel shell wall at 2 m level (0°) | S-8a |  |
| Vessel shell wall at 2 m level (90°) | S-8b |  |
| Vessel shell wall at 2 m level (180°) | S-8c |  |
| Vessel shell wall at 2 m level (270°) | S-8d |  |
| Vessel shell wall at 3.5 m level (0°) | S-9a |  |
| Vessel shell wall at 3.5 m level (90°) | S-9b |  |
| Vessel shell wall at 3.5 m level (180°) | S-9c |  |
| Vessel shell wall at 3.5 m level (270°) | S-9d |  |
| Limit |  | NMT 10 ppm |
| LOQ |  | NMT 5 ppm |
| LOD |  | NMT 2 ppm |

    - Rinse samples analysis results summary (Extraneous matter):

As part of the cleaning validation study, final rinse samples shall be evaluated for extraneous matter to confirm that the cleaning process effectively removes visible foreign contaminants from product-contact surfaces. The examination shall include assessment for black particles, fibers, and other extraneous matter. The results from all validation runs are presented in the following table and shall be evaluated against the acceptance criterion that no black or fiber particles are observed in the rinse samples.

| Sample description / location | Sample ID | Results (Extraneous matter) |
| --- | --- | --- |
|  |  | B. No. 3ISMPP4261001 |
| Rinse Sample | NA |  |
| Limit |  | Black and fiber particles should be absent |

Inference:

It shall be written in the cleaning verification report.

Conclusion:

It shall be written in the cleaning verification report.

# PLUG FLOW REACTOR (PFR-1301):

The subject equipment is located in Production Block-2, a multipurpose manufacturing facility. The equipment is qualified for its intended use, and the cleaning validation approach, including sampling locations, has been established based on the approved FMEA (Ref. No. FMEA/PFR-1301-00). This equipment is used in ISM4 manufacturing for nitration reaction between ISM3 and fuming nitric acid to obtain ISM4A.

    - Equipment details

The equipment details, including Material of Construction (MOC), product contact surface area, shell height, and shell diameter, were obtained from CPDR Annexure-2 and the applicable equipment qualification documents. These parameters were used to establish the shell wall swab sampling locations.

| Parameter | Details | Reference |
| --- | --- | --- |
| Capacity | 5 L | CPDR Annexure-2 |
| MOC | SS 316L | CPDR Annexure-2 |
| Surface Area | 0.0785 m2 | CPDR Annexure-2 |
| Shell length | 2400 mm | CPDR Annexure-2 |
| Shell diameter |  | CPDR Annexure-2 |

    - Supporting Documents and References

| Documents | Document # | Effective / Approval date |
| --- | --- | --- |
| BCR | PB2-BCR-ISM4-XXX-00 |  |
| Specification | QCD-CLS-ISM4A-001-00 | 15-09-2025 |
| Testing Procedure | QCD-CLT-ISMC-001-00 | 15-09-2025 |
| Analytical Method Validation | IMN-VAL-FP-CLM-25-015 | 15-10-2025 |
| Equipment Qualification (PQ) | PQR/PFR-1301-00 |  |

    - Cleaning validation results summary:

Upon completion of cleaning, rinse sample shall be collected from the equipment. The results of rinse samples shall be evaluated against the established acceptance criteria to demonstrate the effectiveness and consistency of the cleaning procedure.

    - Rinse samples analysis results summary (ISM4A content):

The following table summarizes the sampling locations and results for ISM4A residue analysis during the cleaning validation study. Results from the verification run shall be compared against the established acceptance criterion of NMT 10 ppm.

| Sample description / location | Sample ID | Results (ISM4A content) |
| --- | --- | --- |
|  |  | B. No. 3ISMPP4261001 |
| Final Rinse Sample (Acetone) | NA |  |
| Limit |  | NMT 10 ppm |
| LOQ |  | NMT 5 ppm |
| LOD |  | NMT 2 ppm |

    - Rinse samples analysis results summary (Extraneous matter):

As part of the cleaning validation study, final rinse samples shall be evaluated for extraneous matter to confirm that the cleaning process effectively removes visible foreign contaminants from product-contact surfaces. The examination shall include assessment for black particles, fibers, and other extraneous matter. The results from all validation runs are presented in the following table and shall be evaluated against the acceptance criterion that no black or fiber particles are observed in the rinse samples.

| Sample description / location | Sample ID | Results (Extraneous matter) |
| --- | --- | --- |
|  |  | B. No. 3ISMPP4261001 |
| Rinse Sample | NA |  |
| Limit |  | Black and fiber particles should be absent |

Inference:

It shall be written in the cleaning verification report.

Conclusion:

It shall be written in the cleaning verification report.

# GLASS LINED REACTOR (GLR-1305):

The subject equipment is located in Production Block-2, a multipurpose manufacturing facility. The equipment is qualified for its intended use, and the cleaning validation approach, including sampling locations, has been established based on the approved FMEA (Ref. No. FMEA/GLR-1305-00). This is equipment is being used for quenching and extraction of ISM4A product.

    - Equipment details

The equipment details, including Material of Construction (MOC), product contact surface area, shell height, and shell diameter, were obtained from CPDR Annexure-2 and the applicable equipment qualification documents. These parameters were used to establish the shell wall swab sampling locations.

| Parameter | Details | Reference |
| --- | --- | --- |
| Capacity | 10 KL | CPDR Annexure-2 |
| MOC | MSGL | CPDR Annexure-2 |
| Surface Area | 30.96 m2 | CPDR Annexure-2 |
| Shell height | 2598 mm | CPDR Annexure-2 |
| Shell diameter | 2465 mm | CPDR Annexure-2 |
| Type of agitator | RCBT+PBT | IQ report#: IQR/GLR-1305-00 |

    - Supporting Documents and References

| Documents | Document # | Effective / Approval date |
| --- | --- | --- |
| BCR | PB2-BCR-ISM4-XXX-00 |  |
| Specification | QCD-CLS-ISM4A-001-00 | 15-09-2025 |
| Testing Procedure | QCD-CLT-ISMC-001-00 | 15-09-2025 |
| Analytical Method Validation | IMN-VAL-FP-CLM-25-015 | 15-10-2025 |
| Equipment Qualification (PQ) | PQR/GLR-1305-00 | 17-09-2026 |

    - Swab sampling locations determination:

      - Worst-case locations:

The following worst-case product-contact locations shall be included as mandatory swab sampling locations based on equipment design, accessibility, cleanability, and residue accumulation potential.

| S-1 | Reactor top dish near manhole |
| --- | --- |
| S-2 | Reactor bottom side inner surface |
| S-3 | Manhole lid seating area |
| S-4 | Reactor’s Stirrer middle location |
| S-5 | Reactor’s Stirrer blade |
| S-6 | Bottom discharge valve |
| S-7 | Thermowell bottom |

      - Calculation for shell wall swab locations:

The number of shell wall swab sampling locations has been determined from the vessel shell height and diameter, to ensure representative coverage of the internal product-contact surface. Refer to Section 14.0.

| Parameter | Calculation | Value | Remarks |
| --- | --- | --- | --- |
| Shell height (H) | NA | 2.6 m | Measured from the bottom tangent line to the top tangent line |
| No. of horizontal levels (n = √H+1) | √2.6+1 | 3 #s | Rounded to the nearest whole number. Levels at 0 m, 1.3 m and 2.6 m height |
| Shell Diameter (D) | NA | 2.5 m | D > 1 m, hence the four-location criterion applies |
| No. of circumferential locations (C) Case 1: D<1 = two locations Case 2: D>1 = four locations | NA | 4 #s | Four circumferential locations (0°, 90°, 180° and 270°) at each horizontal level, as D = 2.5 m falls under Case 2 |
| Total No. of samples on wall surface (n × C) | 4 x 3 | 12 #s | Total of 12 swab locations on the shell wall surface |

      - Pictorial representation of GLR-1305:

S-1

S-3

S-2

S-9a to S-9d

(1.3 meter level)

S-4

S-5

S-6

S-10a to S-10d

(2.6 meter level)

S-7

S-8a to S-8d

(0 meter level)

      - Rationale for swab sample locations:

Swab sampling locations have been selected on a risk basis from the equipment design and operation, considering accessibility, cleanability, residue accumulation potential and the extent of product-contact surface. They comprise the mandatory worst-case locations (S-1 to S-7) and representative locations on the vessel shell wall (S-8 to S-10), the number of which is derived in Section 14.0.

| Swab ID. # | Description | ###### Rationale (Based on operation and design of equipment) | ###### No. of samples |
| --- | --- | --- | --- |
| S-1 | Top dish near manhole | Product contact surface Hard to clean (overhead surface, shadowed from direct spray) Potential for splash and residue deposition | 1 |
| S-2 | Bottom side inner surface near discharge | Product contact surface Product accumulation (lowest point) Hard to reach Likely worst-case for residue retention | 1 |
| S-3 | Manhole lid seating area | Hard to clean (gasket and crevice) Seam and joint geometry Potential for residue entrapment | 1 |
| S-4 | Stirrer shaft, middle location | Product contact Hard to reach Irregular shape | 1 |
| S-5 | Stirrer blade | Product contact surface Hard to reach Irregular shape (blade and hub junction) Potential spray shadow | 1 |
| S-6 | Bottom discharge valve | Product contact surface Hard to clean Seam and gasket Irregular shape Lowest point, flow dead-leg potential | 1 |
| S-7 | Thermowell bottom | Product-contact surface with protruding geometry; potential spray-shadow area; limited fluid turbulence around the thermowell may result in localized residue retention and reduced cleanability | 1 |
| S-8a, S-8b, S-8c, S-8d | Shell wall at 0 m level (0°, 90°, 180°, 270°) | Product contact surface Representative of the largest contact area Coverage of the lower shell, where product accumulates | 4 |
| S-9a, S-9b, S-9c,S-9d | Shell wall at 1.3 m level (0°, 90°, 180°, 270°) | Product contact surface Representative of the largest contact area Coverage of the mid shell | 4 |
| S-10a, S-10b, S-10c, S-10d | Shell wall at 2.6 m level (0°, 90°, 180°, 270°) | Product contact surface Representative of the largest contact area Coverage of the upper shell, where splash and vapor condensation can deposit residue | 4 |

    - Cleaning validation results summary:

During the cleaning process, reflux samples shall be collected during the reflux cleaning step and analyzed to assess residue removal efficiency. Upon completion of cleaning, the equipment shall undergo visual inspection under qualified inspection conditions. Following satisfactory visual inspection, swab samples shall be collected from predefined worst-case locations and final rinse samples shall be collected from the equipment. The results of visual inspection and analysis of reflux, swab, and rinse samples shall be evaluated against the established acceptance criteria to demonstrate the effectiveness and consistency of the cleaning procedure.

    - Visual inspection summary

As a primary verification of equipment cleanliness, visual inspection shall be conducted under established inspection conditions to assess the presence of visible product residues, foreign matter, staining, or other indications of inadequate cleaning. The observations made by the Production Chemist, Production Shift In-charge, and QA representative during cleaning verification run are presented in the following table. Any permanent stains observed shall be evaluated to confirm that they do not impact the effectiveness of the cleaning process or product quality.

| Sample description / location | Results |
| --- | --- |
|  | B. No. 3ISMPP4261001 |
| Production Chemist verification |  |
| Production Shift in-charge verification |  |
| QA verification |  |
| Is any permanent stain recorded? |  |

    - Reflux, Swab & Rinse samples analysis results summary (ISM4A content):

The following table summarizes the sampling locations and results for ISM4A residue analysis during the cleaning validation study. Results from all validation runs shall be compared against the established acceptance criterion of NMT 10 ppm.

| Sample description / location | Sample ID | Results (ISM4A content) |
| --- | --- | --- |
|  |  | B. No. 3ISMPP4261001 |
| Rinse Sample (Acetone) after water reflux - for information | NA |  |
| Reflux Sample (Acetone) - for information | NA |  |
| Final Rinse Sample (Acetone) | NA |  |
| Swab samples |  |  |
| Top dish near manhole | S-1 |  |
| Bottom side inner surface | S-2 |  |
| Manhole lid seating area | S-3 |  |
| Stirrer shaft, middle location | S-4 |  |
| Stirrer blade | S-5 |  |
| Bottom discharge valve | S-6 |  |
| Thermowell bottom | S-7 |  |
| Reactor shell’s wall surface at 0-meter level - 0° | S-8a |  |
| Reactor shell’s wall surface at 0-meter level - 90° | S-8b |  |
| Reactor shell’s wall surface at 0-meter level - 180° | S-8c |  |
| Reactor shell’s wall surface at 0-meter level - 270° | S-8d |  |
| Reactor shell’s wall surface at 1.3-meter level - 0° | S-9a |  |
| Reactor shell’s wall surface at 1.3-meter level - 90° | S-9b |  |
| Reactor shell’s wall surface at 1.3-meter level - 180° | S-9c |  |
| Reactor shell’s wall surface at 1.3-meter level - 270° | S-9d |  |
| Reactor shell’s wall surface at 2.6-meter level - 0° | S-10a |  |
| Reactor shell’s wall surface at 2.6-meter level - 90° | S-10b |  |
| Reactor shell’s wall surface at 2.6-meter level - 180° | S-10c |  |
| Reactor shell’s wall surface at 2.6-meter level - 270° | S-10d |  |
| Limit |  | NMT 10 ppm |
| LOQ |  | NMT 5 ppm |
| LOD |  | NMT 2 ppm |

    - Rinse samples analysis results summary (Extraneous matter):

As part of the cleaning verification study, final rinse samples shall be evaluated for extraneous matter to confirm that the cleaning process effectively removes visible foreign contaminants from product-contact surfaces. The examination shall include assessment for black particles, fibers, and other extraneous matter. The results from all validation runs are presented in the following table and shall be evaluated against the acceptance criterion that no black or fiber particles are observed in the rinse samples.

| Sample description / location | Sample ID | Results (Extraneous matter) |
| --- | --- | --- |
|  |  | B. No. 3ISMPP4261001 |
| Rinse Sample | NA |  |
| Limit |  | Black and fiber particles should be absent |

Inference:

It shall be written in the cleaning verification report.

Conclusion:

It shall be written in the cleaning verification report.

# GLASS LINED REACTOR (GLR-1304):

The subject equipment is located in Production Block-2, a multipurpose manufacturing facility. The equipment is qualified for its intended use, and the cleaning validation approach, including sampling locations, has been established based on the approved FMEA (Ref. No. FMEA/GLR-1304-00). This is equipment is being used for quenching and extraction of ISM4A product.

    - Equipment details

The equipment details, including Material of Construction (MOC), product contact surface area, shell height, and shell diameter, were obtained from CPDR Annexure-2 and the applicable equipment qualification documents. These parameters were used to establish the shell wall swab sampling locations.

| Parameter | Details | Reference |
| --- | --- | --- |
| Capacity | 16 KL | CPDR Annexure-2 |
| MOC | MSGL | CPDR Annexure-2 |
| Surface Area | 30.96 m2 | CPDR Annexure-2 |
| Shell height | 3705 mm | CPDR Annexure-2 |
| Shell diameter | 2772 mm | CPDR Annexure-2 |
| Type of agitator | RCBT+PBT | IQ report#: IQR/GLR-1304-00 |

    - Supporting Documents and References

| Documents | Document # | Effective / Approval date |
| --- | --- | --- |
| BCR | PB2-BCR-ISM4-XXX-00 |  |
| Specification | QCD-CLS-ISM4A-001-00 | 15-09-2025 |
| Testing Procedure | QCD-CLT-ISMC-001-00 | 15-09-2025 |
| Analytical Method Validation | IMN-VAL-FP-CLM-25-015 | 15-10-2025 |
| Equipment Qualification (PQ) | PQR/GLR-1304-00 | 17-09-2026 |

    - Swab sampling locations determination:

      - Worst-case locations:

The following worst-case product-contact locations shall be included as mandatory swab sampling locations based on equipment design, accessibility, cleanability, and residue accumulation potential.

| S-1 | Reactor top dish near manhole |
| --- | --- |
| S-2 | Reactor bottom side inner surface |
| S-3 | Manhole lid seating area |
| S-4 | Reactor’s Stirrer middle location |
| S-5 | Reactor’s Stirrer blade |
| S-6 | Bottom discharge valve |
| S-7 | Thermowell bottom |

      - Calculation for shell wall swab locations:

The number of shell wall swab sampling locations has been determined from the vessel shell height and diameter, to ensure representative coverage of the internal product-contact surface. Refer to Section 14.0.

| Parameter | Calculation | Value | Remarks |
| --- | --- | --- | --- |
| Shell height (H) | NA | 3.7 m | Measured from the bottom tangent line to the top tangent line |
| No. of horizontal levels (n = √H+1) | √3.7+1 | 3 #s | Rounded to the nearest whole number. Levels at 0 m, 2 m and 3.7 m height |
| Shell Diameter (D) | NA | 2.7 m | D > 1 m, hence the four-location criterion applies |
| No. of circumferential locations (C) Case 1: D<1 = two locations Case 2: D>1 = four locations | NA | 4 #s | Four circumferential locations (0°, 90°, 180° and 270°) at each horizontal level, as D = 2.7 m falls under Case 2 |
| Total No. of samples on wall surface (n × C) | 4 x 3 | 12 #s | Total of 12 swab locations on the shell wall surface |

      - Pictorial representation of GLR-1304:

S-1

S-3

S-2

S-9a to S-9d

(2 meter level)

S-4

S-5

S-6

S-10a to S-10d

(3.7 meter level)

S-7

S-8a to S-8d

(0 meter level)

      - Rationale for swab sample locations:

Swab sampling locations have been selected on a risk basis from the equipment design and operation, considering accessibility, cleanability, residue accumulation potential and the extent of product-contact surface. They comprise the mandatory worst-case locations (S-1 to S-7) and representative locations on the vessel shell wall (S-8 to S-10), the number of which is derived in Section 14.0.

| Swab ID. # | Description | ###### Rationale (Based on operation and design of equipment) | ###### No. of samples |
| --- | --- | --- | --- |
| S-1 | Top dish near manhole | Product contact surface Hard to clean (overhead surface, shadowed from direct spray) Potential for splash and residue deposition | 1 |
| S-2 | Bottom side inner surface near discharge | Product contact surface Product accumulation (lowest point) Hard to reach Likely worst-case for residue retention | 1 |
| S-3 | Manhole lid seating area | Hard to clean (gasket and crevice) Seam and joint geometry Potential for residue entrapment | 1 |
| S-4 | Stirrer shaft, middle location | Product contact Hard to reach Irregular shape | 1 |
| S-5 | Stirrer blade | Product contact surface Hard to reach Irregular shape (blade and hub junction) Potential spray shadow | 1 |
| S-6 | Bottom discharge valve | Product contact surface Hard to clean Seam and gasket Irregular shape Lowest point, flow dead-leg potential | 1 |
| S-7 | Thermowell bottom | Product-contact surface with protruding geometry; potential spray-shadow area; limited fluid turbulence around the thermowell may result in localized residue retention and reduced cleanability | 1 |
| S-8a, S-8b, S-8c, S-8d | Shell wall at 0 m level (0°, 90°, 180°, 270°) | Product contact surface Representative of the largest contact area Coverage of the lower shell, where product accumulates | 4 |
| S-9a, S-9b, S-9c,S-9d | Shell wall at 2 m level (0°, 90°, 180°, 270°) | Product contact surface Representative of the largest contact area Coverage of the mid shell | 4 |
| S-10a, S-10b, S-10c, S-10d | Shell wall at 3.7 m level (0°, 90°, 180°, 270°) | Product contact surface Representative of the largest contact area Coverage of the upper shell, where splash and vapor condensation can deposit residue | 4 |

    - Cleaning validation results summary:

During the cleaning process, reflux samples shall be collected during the reflux cleaning step and analyzed to assess residue removal efficiency. Upon completion of cleaning, the equipment shall undergo visual inspection under qualified inspection conditions. Following satisfactory visual inspection, swab samples shall be collected from predefined worst-case locations and final rinse samples shall be collected from the equipment. The results of visual inspection and analysis of reflux, swab, and rinse samples shall be evaluated against the established acceptance criteria to demonstrate the effectiveness and consistency of the cleaning procedure.

    - Visual inspection summary

As a primary verification of equipment cleanliness, visual inspection shall be conducted under established inspection conditions to assess the presence of visible product residues, foreign matter, staining, or other indications of inadequate cleaning. The observations made by the Production Chemist, Production Shift In-charge, and QA representative during cleaning verification run are presented in the following table. Any permanent stains observed shall be evaluated to confirm that they do not impact the effectiveness of the cleaning process or product quality.

| Sample description / location | Results |
| --- | --- |
|  | B. No. 3ISMPP4261001 |
| Production Chemist verification |  |
| Production Shift in-charge verification |  |
| QA verification |  |
| Is any permanent stain recorded? |  |

    - Reflux, Swab & Rinse samples analysis results summary (ISM4A content):

The following table summarizes the sampling locations and results for ISM4A residue analysis during the cleaning validation study. Results from all validation runs shall be compared against the established acceptance criterion of NMT 10 ppm.

| Sample description / location | Sample ID | Results (ISM4A content) |
| --- | --- | --- |
|  |  | B. No. 3ISMPP4261001 |
| Rinse Sample (Acetone) after water reflux - for information | NA |  |
| Reflux Sample (Acetone) - for information | NA |  |
| Final Rinse Sample (Acetone) | NA |  |
| Swab samples |  |  |
| Top dish near manhole | S-1 |  |
| Bottom side inner surface | S-2 |  |
| Manhole lid seating area | S-3 |  |
| Stirrer shaft, middle location | S-4 |  |
| Stirrer blade | S-5 |  |
| Bottom discharge valve | S-6 |  |
| Thermowell bottom | S-7 |  |
| Reactor shell’s wall surface at 0-meter level - 0° | S-8a |  |
| Reactor shell’s wall surface at 0-meter level - 90° | S-8b |  |
| Reactor shell’s wall surface at 0-meter level - 180° | S-8c |  |
| Reactor shell’s wall surface at 0-meter level - 270° | S-8d |  |
| Reactor shell’s wall surface at 2-meter level - 0° | S-9a |  |
| Reactor shell’s wall surface at 2-meter level - 90° | S-9b |  |
| Reactor shell’s wall surface at 2-meter level - 180° | S-9c |  |
| Reactor shell’s wall surface at 2-meter level - 270° | S-9d |  |
| Reactor shell’s wall surface at 3.7-meter level - 0° | S-10a |  |
| Reactor shell’s wall surface at 3.7-meter level - 90° | S-10b |  |
| Reactor shell’s wall surface at 3.7-meter level - 180° | S-10c |  |
| Reactor shell’s wall surface at 3.7-meter level - 270° | S-10d |  |
| Limit |  | NMT 10 ppm |
| LOQ |  | NMT 5 ppm |
| LOD |  | NMT 2 ppm |

    - Rinse samples analysis results summary (Extraneous matter):

As part of the cleaning verification study, final rinse samples shall be evaluated for extraneous matter to confirm that the cleaning process effectively removes visible foreign contaminants from product-contact surfaces. The examination shall include assessment for black particles, fibers, and other extraneous matter. The results from all validation runs are presented in the following table and shall be evaluated against the acceptance criterion that no black or fiber particles are observed in the rinse samples.

| Sample description / location | Sample ID | Results (Extraneous matter) |
| --- | --- | --- |
|  |  | B. No. 3ISMPP4261001 |
| Rinse Sample | NA |  |
| Limit |  | Black and fiber particles should be absent |

Inference:

It shall be written in the cleaning verification report.

Conclusion:

It shall be written in the cleaning verification report.

# GLASS LINED REACTOR (GLR-1303):

The subject equipment is located in Production Block-2, a multipurpose manufacturing facility. The equipment is qualified for its intended use, and the cleaning validation approach, including sampling locations, has been established based on the approved FMEA (Ref. No. FMEA/GLR-1303-00). This is equipment is being used for reaction to obtain ISM4 from ISM4A.

    - Equipment details

The equipment details, including Material of Construction (MOC), product contact surface area, shell height, and shell diameter, were obtained from CPDR Annexure-2 and the applicable equipment qualification documents. These parameters were used to establish the shell wall swab sampling locations.

| Parameter | Details | Reference |
| --- | --- | --- |
| Capacity | 25 KL | CPDR Annexure-2 |
| MOC | MSGL | CPDR Annexure-2 |
| Surface Area | 30.96 m2 | CPDR Annexure-2 |
| Shell height | 4755 mm | CPDR Annexure-2 |
| Shell diameter | 2986 mm | CPDR Annexure-2 |
| Type of agitator | RCBT+PBT | IQ report#: IQR/GLR-1303-00 |

    - Supporting Documents and References

| Documents | Document # | Effective / Approval date |
| --- | --- | --- |
| BCR | PB2-BCR-ISM4-XXX-00 |  |
| Specification | QCD-CLS-ISM-001-00 | 27-11-2025 |
| Testing Procedure | QCD-CLT-ISM-001-00 | 27-11-2025 |
| Analytical Method Validation | PR-MV-26-0011 | 22-09-2026 |
| Equipment Qualification (PQ) | PQR/GLR-1303-00 | 18-09-2026 |

    - Swab sampling locations determination:

      - Worst-case locations:

The following worst-case product-contact locations shall be included as mandatory swab sampling locations based on equipment design, accessibility, cleanability, and residue accumulation potential.

| S-1 | Reactor top dish near manhole |
| --- | --- |
| S-2 | Reactor bottom side inner surface |
| S-3 | Manhole lid seating area |
| S-4 | Reactor’s Stirrer middle location |
| S-5 | Reactor’s Stirrer blade |
| S-6 | Bottom discharge valve |
| S-7 | Thermowell bottom |

      - Calculation for shell wall swab locations:

The number of shell wall swab sampling locations has been determined from the vessel shell height and diameter, to ensure representative coverage of the internal product-contact surface. Refer to Section 14.0.

| Parameter | Calculation | Value | Remarks |
| --- | --- | --- | --- |
| Shell height (H) | NA | 4.8 m | Measured from the bottom tangent line to the top tangent line |
| No. of horizontal levels (n = √H+1) | √4.8+1 | 3 #s | Rounded to the nearest whole number. Levels at 0 m, 2 m and 3.7 m height |
| Shell Diameter (D) | NA | 3 m | D > 1 m, hence the four-location criterion applies |
| No. of circumferential locations (C) Case 1: D<1 = two locations Case 2: D>1 = four locations | NA | 4 #s | Four circumferential locations (0°, 90°, 180° and 270°) at each horizontal level, as D = 3 m falls under Case 2 |
| Total No. of samples on wall surface (n × C) | 4 x 3 | 12 #s | Total of 12 swab locations on the shell wall surface |

      - Pictorial representation of GLR-1303:

S-1

S-3

S-2

S-9a to S-9d

(2.4 meter level)

S-4

S-5

S-6

S-10a to S-10d

(4.8 meter level)

S-7

S-8a to S-8d

(0 meter level)

      - Rationale for swab sample locations:

Swab sampling locations have been selected on a risk basis from the equipment design and operation, considering accessibility, cleanability, residue accumulation potential and the extent of product-contact surface. They comprise the mandatory worst-case locations (S-1 to S-7) and representative locations on the vessel shell wall (S-8 to S-10), the number of which is derived in Section 14.0.

| Swab ID. # | Description | ###### Rationale (Based on operation and design of equipment) | ###### No. of samples |
| --- | --- | --- | --- |
| S-1 | Top dish near manhole | Product contact surface Hard to clean (overhead surface, shadowed from direct spray) Potential for splash and residue deposition | 1 |
| S-2 | Bottom side inner surface near discharge | Product contact surface Product accumulation (lowest point) Hard to reach Likely worst-case for residue retention | 1 |
| S-3 | Manhole lid seating area | Hard to clean (gasket and crevice) Seam and joint geometry Potential for residue entrapment | 1 |
| S-4 | Stirrer shaft, middle location | Product contact Hard to reach Irregular shape | 1 |
| S-5 | Stirrer blade | Product contact surface Hard to reach Irregular shape (blade and hub junction) Potential spray shadow | 1 |
| S-6 | Bottom discharge valve | Product contact surface Hard to clean Seam and gasket Irregular shape Lowest point, flow dead-leg potential | 1 |
| S-7 | Thermowell bottom | Product-contact surface with protruding geometry; potential spray-shadow area; limited fluid turbulence around the thermowell may result in localized residue retention and reduced cleanability | 1 |
| S-8a, S-8b, S-8c, S-8d | Shell wall at 0 m level (0°, 90°, 180°, 270°) | Product contact surface Representative of the largest contact area Coverage of the lower shell, where product accumulates | 4 |
| S-9a, S-9b, S-9c,S-9d | Shell wall at 2.4 m level (0°, 90°, 180°, 270°) | Product contact surface Representative of the largest contact area Coverage of the mid shell | 4 |
| S-10a, S-10b, S-10c, S-10d | Shell wall at 4.8 m level (0°, 90°, 180°, 270°) | Product contact surface Representative of the largest contact area Coverage of the upper shell, where splash and vapor condensation can deposit residue | 4 |

    - Cleaning validation results summary:

During the cleaning process, reflux samples shall be collected during the reflux cleaning step and analyzed to assess residue removal efficiency. Upon completion of cleaning, the equipment shall undergo visual inspection under qualified inspection conditions. Following satisfactory visual inspection, swab samples shall be collected from predefined worst-case locations and final rinse samples shall be collected from the equipment. The results of visual inspection and analysis of reflux, swab, and rinse samples shall be evaluated against the established acceptance criteria to demonstrate the effectiveness and consistency of the cleaning procedure.

    - Visual inspection summary

As a primary verification of equipment cleanliness, visual inspection shall be conducted under established inspection conditions to assess the presence of visible product residues, foreign matter, staining, or other indications of inadequate cleaning. The observations made by the Production Chemist, Production Shift In-charge, and QA representative during cleaning verification run are presented in the following table. Any permanent stains observed shall be evaluated to confirm that they do not impact the effectiveness of the cleaning process or product quality.

| Sample description / location | Results |
| --- | --- |
|  | B. No. 3ISMPP4261001 |
| Production Chemist verification |  |
| Production Shift in-charge verification |  |
| QA verification |  |
| Is any permanent stain recorded? |  |

    - Reflux, Swab & Rinse samples analysis results summary (ISM4 content):

The following table summarizes the sampling locations and results for ISM4 residue analysis during the cleaning validation study. Results from all validation runs shall be compared against the established acceptance criterion of NMT 10 ppm.

| Sample description / location | Sample ID | Results (ISM4A content) |
| --- | --- | --- |
|  |  | B. No. 3ISMPP4261001 |
| Reflux Sample (Water) - for information | NA |  |
| Reflux Sample (Acetone) - for information | NA |  |
| Final Rinse Sample (Acetone) | NA |  |
| Swab samples |  |  |
| Top dish near manhole | S-1 |  |
| Bottom side inner surface | S-2 |  |
| Manhole lid seating area | S-3 |  |
| Stirrer shaft, middle location | S-4 |  |
| Stirrer blade | S-5 |  |
| Bottom discharge valve | S-6 |  |
| Thermowell bottom | S-7 |  |
| Reactor shell’s wall surface at 0-meter level - 0° | S-8a |  |
| Reactor shell’s wall surface at 0-meter level - 90° | S-8b |  |
| Reactor shell’s wall surface at 0-meter level - 180° | S-8c |  |
| Reactor shell’s wall surface at 0-meter level - 270° | S-8d |  |
| Reactor shell’s wall surface at 2.4-meter level - 0° | S-9a |  |
| Reactor shell’s wall surface at 2.4-meter level - 90° | S-9b |  |
| Reactor shell’s wall surface at 2.4-meter level - 180° | S-9c |  |
| Reactor shell’s wall surface at 2.4-meter level - 270° | S-9d |  |
| Reactor shell’s wall surface at 4.8-meter level - 0° | S-10a |  |
| Reactor shell’s wall surface at 4.8-meter level - 90° | S-10b |  |
| Reactor shell’s wall surface at 4.8-meter level - 180° | S-10c |  |
| Reactor shell’s wall surface at 4.8-meter level - 270° | S-10d |  |
| Limit |  | NMT 10 ppm |
| LOQ |  | NMT 1 ppm |
| LOD |  | NMT 0.3 ppm |

    - Rinse samples analysis results summary (Extraneous matter):

As part of the cleaning verification study, final rinse samples shall be evaluated for extraneous matter to confirm that the cleaning process effectively removes visible foreign contaminants from product-contact surfaces. The examination shall include assessment for black particles, fibers, and other extraneous matter. The results from all validation runs are presented in the following table and shall be evaluated against the acceptance criterion that no black or fiber particles are observed in the rinse samples.

| Sample description / location | Sample ID | Results (Extraneous matter) |
| --- | --- | --- |
|  |  | B. No. 3ISMPP4261001 |
| Rinse Sample | NA |  |
| Limit |  | Black and fiber particles should be absent |

Inference:

It shall be written in the cleaning verification report.

Conclusion:

It shall be written in the cleaning verification report.

# STAINLESS STEEL REACTOR (SSR-1305):

The subject equipment is located in Production Block-2, a multipurpose manufacturing facility. The equipment is qualified for its intended use, and the cleaning validation approach, including sampling locations, has been established based on the approved FMEA (Ref. No. FMEA/SSR-1305-00). This is equipment is being used for distillation and crystallization of the product.

    - Equipment details

The equipment details, including Material of Construction (MOC), product contact surface area, shell height, and shell diameter, were obtained from CPDR Annexure-2 and the applicable equipment qualification documents. These parameters were used to establish the shell wall swab sampling locations.

| Parameter | Details | Reference |
| --- | --- | --- |
| Capacity | 10 KL | CPDR Annexure-2 |
| MOC | SS 316L | CPDR Annexure-2 |
| Surface Area | 30.5 m2 | CPDR Annexure-2 |
| Shell height | 3457 mm | CPDR Annexure-2 |
| Shell diameter | 2100 mm | CPDR Annexure-2 |
| Type of agitator | Anchor | IQ report#: IQR/SSR-1305-00 |
| No. of baffles | 4 | IQ report#: IQR/SSR-1305-00 |

    - Supporting Documents and References

| Documents | Document # | Effective / Approval date |
| --- | --- | --- |
| BCR | PB2-BCR-ISM4-XXX-00 |  |
| Specification | QCD-CLS-ISM-001-00 | 27-11-2025 |
| Testing Procedure | QCD-CLT-ISM-001-00 | 27-11-2025 |
| Analytical Method Validation | PR-MV-26-0011 | 22-09-2026 |
| Equipment Qualification (PQ) | PQR/SSR-1305-00 | 11-09-2026 |

    - Swab sampling locations determination:

      - Worst-case locations:

The following worst-case product-contact locations shall be included as mandatory swab sampling locations based on equipment design, accessibility, cleanability, and residue accumulation potential.

| S-1 | Reactor top dish near manhole |
| --- | --- |
| S-2 | Reactor bottom side inner surface |
| S-3 | Manhole lid seating area |
| S-4 | Reactor’s Stirrer middle location |
| S-5 | Reactor’s Stirrer blade |
| S-6 | Bottom discharge valve |
| S-7 | Thermowell bottom |

      - Calculation for shell wall swab locations:

The number of shell wall swab sampling locations has been determined from the vessel shell height and diameter, to ensure representative coverage of the internal product-contact surface. Refer to Section 14.0.

| Parameter | Calculation | Value | Remarks |
| --- | --- | --- | --- |
| Shell height (H) | NA | 3.5 m | Measured from the bottom tangent line to the top tangent line |
| No. of horizontal levels (n = √H+1) | √3.5+1 | 3 #s | Rounded to the nearest whole number. Levels at 0 m, 2 m and 3.5 m height |
| Shell Diameter (D) | NA | 2.1 m | D > 1 m, hence the four-location criterion applies |
| No. of circumferential locations (C) Case 1: D<1 = two locations Case 2: D>1 = four locations | NA | 4 #s | Four circumferential locations (0°, 90°, 180° and 270°) at each horizontal level, as D = 2.1 m falls under Case 2 |
| Total No. of samples on wall surface (n × C) | 4 x 3 | 12 #s | Total of 12 swab locations on the shell wall surface |

      - Pictorial representation of SSR-1305:

S-1

S-3

S-2

S-4a to S-4d

(2 meter level)

S-5

S-6

S-7

S-4e to S-4h

(3.5 meter level)

      - Rationale for swab sample locations:

Swab sampling locations have been selected on a risk basis from the equipment design and operation, considering accessibility, cleanability, residue accumulation potential and the extent of product-contact surface. They comprise the mandatory worst-case locations (S-1 to S-7) and representative locations on the vessel shell wall (S-8 to S-10), the number of which is derived in Section 14.0.

| Swab ID. # | Description | ###### Rationale (Based on operation and design of equipment) | ###### No. of samples |
| --- | --- | --- | --- |
| S-1 | Top dish near manhole | Product contact surface Hard to clean (overhead surface, shadowed from direct spray) Potential for splash and residue deposition | 1 |
| S-2 | Bottom side inner surface near discharge | Product contact surface Product accumulation (lowest point) Hard to reach Likely worst-case for residue retention | 1 |
| S-3 | Manhole lid seating area | Hard to clean (gasket and crevice) Seam and joint geometry Potential for residue entrapment | 1 |
| S-4 | Stirrer shaft, middle location | Product contact Hard to reach Irregular shape | 1 |
| S-5 | Stirrer blade | Product contact surface Hard to reach Irregular shape (blade and hub junction) Potential spray shadow | 1 |
| S-6 | Bottom discharge valve | Product contact surface Hard to clean Seam and gasket Irregular shape Lowest point, flow dead-leg potential | 1 |
| S-7 | Thermowell bottom | Product-contact surface with protruding geometry; potential spray-shadow area; limited fluid turbulence around the thermowell may result in localized residue retention and reduced cleanability | 1 |
| S-8a, S-8b, S-8c, S-8d | Shell wall at 0 m level (0°, 90°, 180°, 270°) | Product contact surface Representative of the largest contact area Coverage of the lower shell, where product accumulates | 4 |
| S-9a, S-9b, S-9c,S-9d | Shell wall at 2 m level (0°, 90°, 180°, 270°) | Product contact surface Representative of the largest contact area Coverage of the mid shell | 4 |
| S-10a, S-10b, S-10c, S-10d | Shell wall at 3.5 m level (0°, 90°, 180°, 270°) | Product contact surface Representative of the largest contact area Coverage of the upper shell, where splash and vapor condensation can deposit residue | 4 |

    - Cleaning validation results summary:

During the cleaning process, reflux samples shall be collected during the reflux cleaning step and analyzed to assess residue removal efficiency. Upon completion of cleaning, the equipment shall undergo visual inspection under qualified inspection conditions. Following satisfactory visual inspection, swab samples shall be collected from predefined worst-case locations and final rinse samples shall be collected from the equipment. The results of visual inspection and analysis of reflux, swab, and rinse samples shall be evaluated against the established acceptance criteria to demonstrate the effectiveness and consistency of the cleaning procedure.

    - Visual inspection summary

As a primary verification of equipment cleanliness, visual inspection shall be conducted under established inspection conditions to assess the presence of visible product residues, foreign matter, staining, or other indications of inadequate cleaning. The observations made by the Production Chemist, Production Shift In-charge, and QA representative during cleaning verification run are presented in the following table. Any permanent stains observed shall be evaluated to confirm that they do not impact the effectiveness of the cleaning process or product quality.

| Sample description / location | Results |
| --- | --- |
|  | B. No. 3ISMPP4261001 |
| Production Chemist verification |  |
| Production Shift in-charge verification |  |
| QA verification |  |
| Is any permanent stain recorded? |  |

    - Reflux, Swab & Rinse samples analysis results summary (ISM4 content):

The following table summarizes the sampling locations and results for ISM4 residue analysis during the cleaning validation study. Results from all validation runs shall be compared against the established acceptance criterion of NMT 10 ppm.

| Sample description / location | Sample ID | Results (ISM4A content) |
| --- | --- | --- |
|  |  | B. No. 3ISMPP4261001 |
| Reflux Sample (Water) - for information | NA |  |
| Reflux Sample (Acetone) - for information | NA |  |
| Final Rinse Sample (Acetone) | NA |  |
| Swab samples |  |  |
| Top dish near manhole | S-1 |  |
| Bottom side inner surface | S-2 |  |
| Manhole lid seating area | S-3 |  |
| Stirrer shaft, middle location | S-4 |  |
| Stirrer blade | S-5 |  |
| Bottom discharge valve | S-6 |  |
| Thermowell bottom | S-7 |  |
| Reactor shell’s wall surface at 0-meter level - 0° | S-8a |  |
| Reactor shell’s wall surface at 0-meter level - 90° | S-8b |  |
| Reactor shell’s wall surface at 0-meter level - 180° | S-8c |  |
| Reactor shell’s wall surface at 0-meter level - 270° | S-8d |  |
| Reactor shell’s wall surface at 2-meter level - 0° | S-9a |  |
| Reactor shell’s wall surface at 2-meter level - 90° | S-9b |  |
| Reactor shell’s wall surface at 2-meter level - 180° | S-9c |  |
| Reactor shell’s wall surface at 2-meter level - 270° | S-9d |  |
| Reactor shell’s wall surface at 3.5-meter level - 0° | S-10a |  |
| Reactor shell’s wall surface at 3.5-meter level - 90° | S-10b |  |
| Reactor shell’s wall surface at 3.5-meter level - 180° | S-10c |  |
| Reactor shell’s wall surface at 3.5-meter level - 270° | S-10d |  |
| Limit |  | NMT 10 ppm |
| LOQ |  | NMT 1 ppm |
| LOD |  | NMT 0.3 ppm |

    - Rinse samples analysis results summary (Extraneous matter):

As part of the cleaning verification study, final rinse samples shall be evaluated for extraneous matter to confirm that the cleaning process effectively removes visible foreign contaminants from product-contact surfaces. The examination shall include assessment for black particles, fibers, and other extraneous matter. The results from all validation runs are presented in the following table and shall be evaluated against the acceptance criterion that no black or fiber particles are observed in the rinse samples.

| Sample description / location | Sample ID | Results (Extraneous matter) |
| --- | --- | --- |
|  |  | B. No. 3ISMPP4261001 |
| Rinse Sample | NA |  |
| Limit |  | Black and fiber particles should be absent |

Inference:

It shall be written in the cleaning verification report.

Conclusion:

It shall be written in the cleaning verification report.

# AGITATED NUTSCHE FILTER (ANF-1301):

The subject equipment is located in Production Block-2, a multipurpose manufacturing facility. The equipment is qualified for its intended use, and the cleaning validation approach, including sampling locations, has been established based on the approved FMEA (Ref. No. FMEA/ANF-1301-00). This is equipment is being used for salt filtration from the product filtrate.

    - Equipment details

The equipment details, including Material of Construction (MOC), product contact surface area, shell height, and shell diameter, were obtained from CPDR Annexure-2 and the applicable equipment qualification documents. These parameters were used to establish the shell wall swab sampling locations.

| Parameter | Details | Reference |
| --- | --- | --- |
| ANF-1301 |  |  |
| Capacity | 3 KL | CPDR Annexure-2 |
| MOC | SS 316 | CPDR Annexure-2 |
| Surface Area | 3.14 m2 | CPDR Annexure-2 |
| Shell height | 1000 mm | CPDR Annexure-2 |
| Shell diameter | 2000 mm | CPDR Annexure-2 |
| ML tank (MLT-1307) |  |  |
| Capacity | 1 KL | CPDR Annexure-2 |
| MOC | SS 316 | CPDR Annexure-2 |
| Surface Area | 6.46 m2 | CPDR Annexure-2 |
| Shell height | 1494 mm | CPDR Annexure-2 |
| Shell diameter | 1000 mm | CPDR Annexure-2 |

    - Supporting Documents and References

| Documents | Document # | Effective / Approval date |
| --- | --- | --- |
| BCR | PB2-BCR-ISM4-XXX-00 |  |
| Specification | QCD-CLS-ISM-001-00 | 27-11-2025 |
| Testing Procedure | QCD-CLT-ISM-001-00 | 27-11-2025 |
| Analytical Method Validation | PR-MV-26-0011 | 22-09-2026 |
| Equipment Qualification (PQ) | PQR/ANF-1301-00 | 11-09-2026 |

    - Swab sampling locations determination:

      - Worst-case locations:

The following worst-case product-contact locations shall be included as mandatory swab sampling locations based on equipment design, accessibility, cleanability, and residue accumulation potential.

| S-1 | ANF agitator |
| --- | --- |
| S-2 | ANF filter mesh |
| S-3 | ANF discharge port |
| S-4 | ANF ML tank inner surface |

      - Calculation for shell wall swab locations:

The number of shell wall swab sampling locations has been determined from the vessel shell height and diameter, to ensure representative coverage of the internal product-contact surface. Refer to Section 14.0.

| Parameter | Calculation | Value | Remarks |
| --- | --- | --- | --- |
| Shell height (H) | NA | 1 m | Measured from the bottom tangent line to the top tangent line |
| No. of horizontal levels | 1 | 1 #s | Height is <2 m, hence one horizontal level is considered. |
| Shell Diameter (D) | NA | 2 m | D > 1 m, hence the four-location criterion applies |
| No. of circumferential locations (C) Case 1: D<1 = two locations Case 2: D>1 = four locations | NA | 4 #s | Four circumferential locations (0°, 90°, 180° and 270°) at each horizontal level, as D = 2 m falls under Case 2 |
| Total No. of samples on wall surface (n × C) | 4 x 1 | 4 #s | Total of 4 swab locations on the shell wall surface |

      - Pictorial representation of ANF-1301:

S-5a to S-5d

S-2

S-3

S-1

      - Rationale for swab sample locations:

Swab sampling locations have been selected on a risk basis from the equipment design and operation, considering accessibility, cleanability, residue accumulation potential and the extent of product-contact surface. They comprise the mandatory worst-case locations (S-1 to S-4) and representative locations on the vessel shell wall (S-5a to S-5d), the number of which is derived in Section 14.0.

| Swab ID. # | Description | ###### Rationale (Based on operation and design of equipment) | ###### No. of samples |
| --- | --- | --- | --- |
| S-1 | ANF agitator | Product-contact surface with complex geometry, welds, and junctions. The agitator is partially shielded during cleaning and may create spray-shadow areas, making it a potential worst-case location for residue retention. | 1 |
| S-2 | ANF filter mesh | Critical product-contact filtration surface directly exposed to the product cake. The mesh contains fine openings and interstitial spaces that may retain product residues, making it one of the most difficult-to-clean locations. | 1 |
| S-3 | ANF discharge port | Product-contact surface containing crevices, seals, and changes in geometry. Acts as the primary discharge pathway and may accumulate residual product during filtration and unloading operations. | 1 |
| S-4 | ANF ML tank inner surface | Product-contact surface receiving filtrate during operation. May be difficult to access for manual inspection and cleaning, with potential for residue deposition and accumulation in low-flow regions. | 1 |
| S-5a to S-5d | Shell wall at middle level (0°, 90°, 180°, 270°) | Representative locations covering the largest product-contact surface area of the ANF vessel. Circumferential sampling provides verification of cleaning effectiveness across the vessel wall and assesses potential residue deposition from filtration, agitation, and slurry contact. | 4 |

    - Cleaning validation results summary:

Upon completion of cleaning, the equipment shall undergo visual inspection under qualified inspection conditions. Following satisfactory visual inspection, swab samples shall be collected from predefined worst-case locations and final rinse samples shall be collected from the equipment. The results of visual inspection and analysis of swab, and rinse samples shall be evaluated against the established acceptance criteria to demonstrate the effectiveness and consistency of the cleaning procedure.

    - Visual inspection summary

As a primary verification of equipment cleanliness, visual inspection shall be conducted under established inspection conditions to assess the presence of visible product residues, foreign matter, staining, or other indications of inadequate cleaning. The observations made by the Production Chemist, Production Shift In-charge, and QA representative during cleaning verification run are presented in the following table. Any permanent stains observed shall be evaluated to confirm that they do not impact the effectiveness of the cleaning process or product quality.

| Sample description / location | Results |
| --- | --- |
|  | B. No. 3ISMPP4261001 |
| Production Chemist verification |  |
| Production Shift in-charge verification |  |
| QA verification |  |
| Is any permanent stain recorded? |  |

    - Swab & Rinse samples analysis results summary (ISM4 content):

The following table summarizes the sampling locations and results for ISM4 residue analysis during the cleaning verification study. Results from all validation runs shall be compared against the established acceptance criterion of NMT 10 ppm.

| Sample description / location | Sample ID | Results (ISM4 content) |
| --- | --- | --- |
|  |  | B. No. 3ISMPP4261001 |
| Final Rinse Sample (Acetone) | NA |  |
| Swab samples |  |  |
| ANF agitator | S-1 |  |
| ANF filter mesh | S-2 |  |
| ANF discharge port | S-3 |  |
| ANF ML tank inner surface | S-4 |  |
| ANF inner shell surface - 0° | S-5a |  |
| ANF inner shell surface - 90° | S-5b |  |
| ANF inner shell surface - 180° | S-5c |  |
| ANF inner shell surface - 270° | S-5d |  |
| Limit |  | NMT 10 ppm |
| LOQ |  | NMT 1 ppm |
| LOD |  | NMT 0.3 ppm |

    - Rinse samples analysis results summary (Extraneous matter):

As part of the cleaning verification study, final rinse samples shall be evaluated for extraneous matter to confirm that the cleaning process effectively removes visible foreign contaminants from product-contact surfaces. The examination shall include assessment for black particles, fibers, and other extraneous matter. The results from all validation runs are presented in the following table and shall be evaluated against the acceptance criterion that no black or fiber particles are observed in the rinse samples.

| Sample description / location | Sample ID | Results (Extraneous matter) |
| --- | --- | --- |
|  |  | B. No. 3ISMPP4261001 |
| Rinse Sample | NA |  |
| Limit |  | Black and fiber particles should be absent |

Inference:

It shall be written in the cleaning verification report.

Conclusion:

It shall be written in the cleaning verification report.

# LEAF FILTER (LF-1302):

The subject equipment is located in Production Block-2, a multipurpose manufacturing facility. The equipment is qualified for its intended use, and the cleaning validation approach, including sampling locations, has been established based on the approved FMEA (Ref. No. FMEA/LF-1302-00). This is equipment is being used for carbon filtration from the product filtrate.

    - Equipment details

The equipment details, including Material of Construction (MOC), product contact surface area, shell height, and shell diameter, were obtained from CPDR Annexure-2 and the applicable equipment qualification documents. These parameters were used to establish the shell wall swab sampling locations.

| Parameter | Details | Reference |
| --- | --- | --- |
| Capacity | 150 L | CPDR Annexure-2 |
| MOC | SS 316 | CPDR Annexure-2 |
| Surface Area | 5.60 m2 | CPDR Annexure-2 |
| Shell height | 1500 mm | CPDR Annexure-2 |
| Shell diameter | 1000 mm | CPDR Annexure-2 |

    - Supporting Documents and References

| Documents | Document # | Effective / Approval date |
| --- | --- | --- |
| BCR | PB2-BCR-ISM4-XXX-00 |  |
| Specification | QCD-CLS-ISM-001-00 | 27-11-2025 |
| Testing Procedure | QCD-CLT-ISM-001-00 | 27-11-2025 |
| Analytical Method Validation | PR-MV-26-0011 | 22-09-2026 |
| Equipment Qualification (OQ) | PQR/LF-1302-00 | 11-09-2026 |

    - Swab sampling locations determination:

      - Worst-case locations:

The following worst-case product-contact locations shall be included as mandatory swab sampling locations based on equipment design, accessibility, cleanability, and residue accumulation potential.

| Location ID | Description of location |
| --- | --- |
| S-1 | Top dome inner surface near spray shadow area |
| S-2 | Filter leaf center surface |
| S-3 | Filter leaf edge/periphery |
| S-4 | Leaf mesh to frame joint/weld |
| S-5 | Header/manifold connection |
| S-6 | Bottom cone lowest point |
| S-7 | Outlet nozzle |
| S-8 | Bottom discharge valve |

      - Calculation for shell wall swab locations:

The number of shell wall swab sampling locations has been determined from the vessel shell height and diameter, to ensure representative coverage of the internal product-contact surface. Refer to Section 14.0.

| Parameter | Calculation | Value | Remarks |
| --- | --- | --- | --- |
| Shell height (H) | NA | 1.5 m | Measured from the bottom tangent line to the top tangent line |
| No. of horizontal levels | NA | 1 # | Height is <2 m, hence one horizontal level is considered at middle level |
| Shell Diameter (D) | NA | 1 m | D = 1 m, hence the two-location criterion applies |
| No. of circumferential locations (C) Case 1: D<1 = two locations Case 2: D>1 = four locations | NA | 4 #s | Two circumferential locations (0° and 180°) at middle level, as D = 1 m falls under Case 1 |
| Total No. of samples on wall surface (n × C) | 2 x 1 | 2 #s | Total of 2 swab locations on the shell wall surface |

      - Pictorial representation of LF-1302:

The following are the pictorial representation of the swab sampling locations as per identified worst-case product-contact locations;

      - Rationale for swab sample locations:

Swab sampling locations have been selected on a risk basis from the equipment design and operation, considering accessibility, cleanability, residue accumulation potential and the extent of product-contact surface. They comprise the mandatory worst-case locations (S-1 to S-8) and representative locations on the vessel shell wall (S-9), the number of which is derived in Section 14.0.

| Swab ID. # | Description | ###### Rationale (Based on operation and design of equipment) | ###### No. of samples |
| --- | --- | --- | --- |
| S-1 | Top dome inner surface near spray shadow area | Product-contact surface located in an overhead region that may receive reduced cleaning solution impingement. Susceptible to splash deposition, condensate formation, and residue accumulation in spray-shadow zones. | 1 |
| S-2 | Filter leaf center surface | Primary filtration surface with direct and prolonged product contact during filtration. Potential for retention of fine particles and dried residues within the filter media structure. | 1 |
| S-3 | Filter leaf edge/periphery | Located at the boundary of the filtration surface where product cake may accumulate and cleaning flow may be less effective. Considered a difficult-to-clean area due to geometric transitions. | 1 |
| S-4 | Leaf mesh to frame joint/weld | Critical worst-case location containing welds, joints, and narrow interfaces that may entrap residues. Cleaning effectiveness can be challenged by crevices and surface discontinuities. | 1 |
| S-5 | Header/manifold connection | Product-contact area with complex geometry, internal passages, and weld connections. Potential hold-up point for filtrate and product residues during operation. | 1 |
| S-6 | Bottom cone lowest point | Lowest point of the equipment where solids, slurry, and residual liquids tend to settle during filtration and draining operations. Represents a worst-case residue accumulation zone. | 1 |
| S-7 | Outlet nozzle | Product-contact transfer pathway with reduced accessibility and potential for residue retention due to changes in flow path and geometry. | 1 |
| S-8 | Bottom discharge valve | Critical worst-case location containing seals, internal cavities, and potential dead-leg areas. Product residues may remain trapped following discharge and cleaning operations. | 1 |
| S-9a, S-9b | Internal shell wall | Representative locations covering the major product-contact surface area. Selected to verify cleaning effectiveness across the vessel wall and assess potential residue deposition during filtration operations. | 2 |

    - Cleaning validation results summary:

Upon completion of cleaning, the equipment shall undergo visual inspection under qualified inspection conditions. Following satisfactory visual inspection, swab samples shall be collected from predefined worst-case locations and final rinse samples shall be collected from the equipment. The results of visual inspection and analysis of swab, and rinse samples shall be evaluated against the established acceptance criteria to demonstrate the effectiveness and consistency of the cleaning procedure.

    - Visual inspection summary

As a primary verification of equipment cleanliness, visual inspection shall be conducted under established inspection conditions to assess the presence of visible product residues, foreign matter, staining, or other indications of inadequate cleaning. The observations made by the Production Chemist, Production Shift In-charge, and QA representative during each cleaning validation run are presented in the following table. Any permanent stains observed shall be evaluated to confirm that they do not impact the effectiveness of the cleaning process or product quality.

| Sample description / location | Results |
| --- | --- |
|  | B. No. 3ISMPP4261001 |
| Production Chemist verification |  |
| Production Shift in-charge verification |  |
| QA verification |  |
| Is any permanent stain recorded? |  |

    - Reflux, Swab & Rinse samples analysis results summary (ISM3 content):

The following table summarizes the sampling locations and results for ISM3 residue analysis during the cleaning validation study. Results from all validation runs shall be compared against the established acceptance criterion of NMT 10 ppm.

| Sample description / location | Sample ID | Results (ISM3 content) |
| --- | --- | --- |
|  |  | B. No. 3ISMPP4261001 |
| Final Rinse Sample (Acetone) | NA |  |
| Swab samples |  |  |
| Top dome inner surface near spray shadow area | S-1 |  |
| Filter leaf center surface | S-2 |  |
| Filter leaf edge/periphery | S-3 |  |
| Leaf mesh to frame joint/weld | S-4 |  |
| Header/manifold connection | S-5 |  |
| Bottom cone lowest point | S-6 |  |
| Outlet nozzle | S-7 |  |
| Bottom discharge valve | S-8 |  |
| Internal shell wall at middle level (0°) | S-9a |  |
| Internal shell wall at middle level (180°) | S-9b |  |
| Limit |  | NMT 10 ppm |
| LOQ |  | NMT 1 ppm |
| LOD |  | NMT 0.3 ppm |

    - Rinse samples analysis results summary (Extraneous matter):

As part of the cleaning validation study, final rinse samples shall be evaluated for extraneous matter to confirm that the cleaning process effectively removes visible foreign contaminants from product-contact surfaces. The examination shall include assessment for black particles, fibers, and other extraneous matter. The results from all validation runs are presented in the following table and shall be evaluated against the acceptance criterion that no black or fiber particles are observed in the rinse samples.

| Sample description / location | Sample ID | Results (Extraneous matter) |
| --- | --- | --- |
|  |  | B. No. 3ISMPP4261001 |
| Rinse Sample | NA |  |
| Limit |  | Black and fiber particles should be absent |

Inference:

It shall be written in the cleaning verification report.

Conclusion:

It shall be written in the cleaning verification report.

# AGITATED NUTSCHE FILTER CUM DRIER (ANFD-1301):

The subject equipment is located in Production Block-2, a multipurpose manufacturing facility. The equipment is qualified for its intended use, and the cleaning validation approach, including sampling locations, has been established based on the approved FMEA (Ref. No. FMEA/ANFD-1301-00). This is equipment is being used for product filtration.

    - Equipment details

The equipment details, including Material of Construction (MOC), product contact surface area, shell height, and shell diameter, were obtained from CPDR Annexure-2 and the applicable equipment qualification documents. These parameters were used to establish the shell wall swab sampling locations.

| Parameter | Details | Reference |
| --- | --- | --- |
| ANFD-1301 |  |  |
| Capacity | 7 KL | CPDR Annexure-2 |
| MOC | SS 316 | CPDR Annexure-2 |
| Surface Area | 7.06 m2 | CPDR Annexure-2 |
| Shell height | 1000 mm | CPDR Annexure-2 |
| Shell diameter | 3000 mm | CPDR Annexure-2 |
| ML tank (MLT-1307) |  |  |
| Capacity | 0.5 KL | CPDR Annexure-2 |
| MOC | SS 316 | CPDR Annexure-2 |
| Surface Area | 3.65 m2 | CPDR Annexure-2 |
| Shell height | 200 mm | CPDR Annexure-2 |
| Shell diameter | 800 mm | CPDR Annexure-2 |

    - Supporting Documents and References

| Documents | Document # | Effective / Approval date |
| --- | --- | --- |
| BCR | PB2-BCR-ISM4-XXX-00 |  |
| Specification | QCD-CLS-ISM-001-00 | 27-11-2025 |
| Testing Procedure | QCD-CLT-ISM-001-00 | 27-11-2025 |
| Analytical Method Validation | PR-MV-26-0011 | 22-09-2026 |
| Equipment Qualification (PQ) | PQR/ANFD-1301-00 | 18-09-2026 |

    - Swab sampling locations determination:

      - Worst-case locations:

The following worst-case product-contact locations shall be included as mandatory swab sampling locations based on equipment design, accessibility, cleanability, and residue accumulation potential.

| S-1 | ANFD agitator |
| --- | --- |
| S-2 | ANFD filter mesh |
| S-3 | ANFD discharge port |
| S-4 | ANFD ML tank inner surface |

      - Calculation for shell wall swab locations:

The number of shell wall swab sampling locations has been determined from the vessel shell height and diameter, to ensure representative coverage of the internal product-contact surface. Refer to Section 14.0.

| Parameter | Calculation | Value | Remarks |
| --- | --- | --- | --- |
| Shell height (H) | NA | 1 m | Measured from the bottom tangent line to the top tangent line |
| No. of horizontal levels | 1 | 1 #s | Height is <2 m, hence one horizontal level is considered. |
| Shell Diameter (D) | NA | 3 m | D > 1 m, hence the four-location criterion applies |
| No. of circumferential locations (C) Case 1: D<1 = two locations Case 2: D>1 = four locations | NA | 4 #s | Four circumferential locations (0°, 90°, 180° and 270°) at each horizontal level, as D = 3 m falls under Case 2 |
| Total No. of samples on wall surface (n × C) | 4 x 1 | 4 #s | Total of 4 swab locations on the shell wall surface |

      - Pictorial representation of ANFD-1301:

S-5a to S-5d

S-2

S-3

S-1

      - Rationale for swab sample locations:

Swab sampling locations have been selected on a risk basis from the equipment design and operation, considering accessibility, cleanability, residue accumulation potential and the extent of product-contact surface. They comprise the mandatory worst-case locations (S-1 to S-4) and representative locations on the vessel shell wall (S-5a to S-5d), the number of which is derived in Section 14.0.

| Swab ID. # | Description | ###### Rationale (Based on operation and design of equipment) | ###### No. of samples |
| --- | --- | --- | --- |
| S-1 | ANFD agitator | Product-contact surface with complex geometry, welds, and junctions. The agitator is partially shielded during cleaning and may create spray-shadow areas, making it a potential worst-case location for residue retention. | 1 |
| S-2 | ANFD filter mesh | Critical product-contact filtration surface directly exposed to the product cake. The mesh contains fine openings and interstitial spaces that may retain product residues, making it one of the most difficult-to-clean locations. | 1 |
| S-3 | ANFD discharge port | Product-contact surface containing crevices, seals, and changes in geometry. Acts as the primary discharge pathway and may accumulate residual product during filtration and unloading operations. | 1 |
| S-4 | ANFD ML tank inner surface | Product-contact surface receiving filtrate during operation. May be difficult to access for manual inspection and cleaning, with potential for residue deposition and accumulation in low-flow regions. | 1 |
| S-5a to S-5d | Shell wall at middle level (0°, 90°, 180°, 270°) | Representative locations covering the largest product-contact surface area of the ANF vessel. Circumferential sampling provides verification of cleaning effectiveness across the vessel wall and assesses potential residue deposition from filtration, agitation, and slurry contact. | 4 |

    - Cleaning validation results summary:

Upon completion of cleaning, the equipment shall undergo visual inspection under qualified inspection conditions. Following satisfactory visual inspection, swab samples shall be collected from predefined worst-case locations and final rinse samples shall be collected from the equipment. The results of visual inspection and analysis of swab, and rinse samples shall be evaluated against the established acceptance criteria to demonstrate the effectiveness and consistency of the cleaning procedure.

    - Visual inspection summary

As a primary verification of equipment cleanliness, visual inspection shall be conducted under established inspection conditions to assess the presence of visible product residues, foreign matter, staining, or other indications of inadequate cleaning. The observations made by the Production Chemist, Production Shift In-charge, and QA representative during cleaning verification run are presented in the following table. Any permanent stains observed shall be evaluated to confirm that they do not impact the effectiveness of the cleaning process or product quality.

| Sample description / location | Results |
| --- | --- |
|  | B. No. 3ISMPP4261001 |
| Production Chemist verification |  |
| Production Shift in-charge verification |  |
| QA verification |  |
| Is any permanent stain recorded? |  |

    - Swab & Rinse samples analysis results summary (ISM4 content):

The following table summarizes the sampling locations and results for ISM4 residue analysis during the cleaning verification study. Results from all validation runs shall be compared against the established acceptance criterion of NMT 10 ppm.

| Sample description / location | Sample ID | Results (ISM4 content) |
| --- | --- | --- |
|  |  | B. No. 3ISMPP4261001 |
| Final Rinse Sample (Acetone) | NA |  |
| Swab samples |  |  |
| ANFD agitator | S-1 |  |
| ANFD filter mesh | S-2 |  |
| ANFD discharge port | S-3 |  |
| ANFD ML tank inner surface | S-4 |  |
| ANFD inner shell surface - 0° | S-5a |  |
| ANFD inner shell surface - 90° | S-5b |  |
| ANFD inner shell surface - 180° | S-5c |  |
| ANFD inner shell surface - 270° | S-5d |  |
| Limit |  | NMT 10 ppm |
| LOQ |  | NMT 1 ppm |
| LOD |  | NMT 0.3 ppm |

    - Rinse samples analysis results summary (Extraneous matter):

As part of the cleaning verification study, final rinse samples shall be evaluated for extraneous matter to confirm that the cleaning process effectively removes visible foreign contaminants from product-contact surfaces. The examination shall include assessment for black particles, fibers, and other extraneous matter. The results from all validation runs are presented in the following table and shall be evaluated against the acceptance criterion that no black or fiber particles are observed in the rinse samples.

| Sample description / location | Sample ID | Results (Extraneous matter) |
| --- | --- | --- |
|  |  | B. No. 3ISMPP4261001 |
| Rinse Sample | NA |  |
| Limit |  | Black and fiber particles should be absent |

Inference:

It shall be written in the cleaning verification report.

Conclusion:

It shall be written in the cleaning verification report.

# NITROSAMINE LIMITS IN THE RINSE SAMPLES:

The rinse samples collected from the equipment after completion of the cleaning procedure shall be analyzed for nitrosamine impurities to verify the effectiveness of the cleaning process and to ensure that nitrosamine residues, if any, are controlled within the predefined acceptance criteria. The rinse samples shall be evaluated for NDMA, NMBA, NDEA, NEIPA, NDIPA, NMPA, and NDBA. The acceptance criterion for each nitrosamine impurity is Not More Than (NMT) 0.01 ppm in the rinse sample. Compliance with the specified limits demonstrates the adequacy of the cleaning procedure in controlling nitrosamine contamination and minimizing the risk of cross-contamination.

| Equipment Name | Equipment ID |  | Nitrosamine impurity limits in Rinse sample (in ppm) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | NDMA | NMBA |  | NDEA |  | NEIPA |  | NDIPA |  | NMPA |  | NDBA |
| Limit NMT (ppm) |  |  | 0.01 | 0.01 |  | 0.01 |  | 0.01 |  | 0.01 |  | 0.01 |  | 0.01 |
| LOD in ppm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| LOQ in ppm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Plug Flow Reactor | PFR-1301 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Glass Lined Reactor | GLR-1305 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Glass Lined Reactor | GLR-1304 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Glass Lined Reactor | GLR-1303 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Glass Lined Reactor | SSR-1305 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ANFD | ANFD-1301 |  |  |  |  |  |  |  |  |  |  |  |  |  |

# POTENTIAL GENOTOXIC IMPURITIES LIMITS IN THE RINSE SAMPLES:

The rinse samples collected from the equipment following completion of the cleaning procedure shall be analyzed for potential genotoxic impurities (PGIs) to verify the effectiveness of the cleaning process and to ensure that any residual PGIs are controlled within the established acceptance criteria. The rinse samples shall be evaluated for O-Nitro Toluene, P-Nitro Toluene, and Mesityl Oxide. The acceptance criterion for each PGI is Not More Than (NMT) 0.2 ppm in the rinse sample. Compliance with the specified limits demonstrates the adequacy of the cleaning procedure in reducing potential genotoxic impurities to acceptable levels and minimizing the risk of cross-contamination in subsequent product manufacture.

| Equipment Name | Equipment ID | PGI limits in Rinse sample (in ppm) |  |  |
| --- | --- | --- | --- | --- |
|  |  | O-Nitro Toluene | P-Nitro Toluene | Mesityl oxide |
| Limit NMT (ppm) |  | 0.2 ppm | 0.2 ppm | 0.2 ppm |
| LOD in ppm |  |  |  |  |
| LOQ in ppm |  |  |  |  |
| Plug Flow Reactor | PFR-1301 |  |  |  |
| Glass Lined Reactor | GLR-1305 |  |  |  |
| Glass Lined Reactor | GLR-1304 |  |  |  |
| Glass Lined Reactor | GLR-1303 |  |  |  |
| Glass Lined Reactor | SSR-1305 |  |  |  |
| ANFD | ANFD-1301 |  |  |  |

# PROCESS LINE CLEANING VERIFICATION SUMMARY

Following completion of equipment cleaning, all associated product-contact process lines, transfer pipelines, flexible hoses, valves, and fittings shall be cleaned as per the approved cleaning procedure. The cleanliness of the process lines shall be verified through visual inspection and, where accessible, white wipe cloth assessment. The verification shall ensure the absence of visible product residues, stains, foreign matter, fibers, black particles, and other extraneous contaminants.

Acceptance Criteria:

No visible product residue, stain, fibers, black particles, or extraneous matter shall be observed.

White wipe cloth used for assessment shall remain visibly clean.

Process lines shall meet established visual cleanliness requirements.

Process Line Cleaning Verification Summary:

| Process Line Description | Production Person Observation | QA Person Observation |
| --- | --- | --- |
| MV-1304 to PFR-1301 Transfer Line |  |  |
| MV-1305 to PFR-1301 Transfer Line |  |  |
| PFR-1301 to GLR-1305 Transfer Line |  |  |
| GLR-1305 to GLR-1304 Transfer Line |  |  |
| GLR-1304 to GLR-1303 Transfer Line |  |  |
| GLR-1303 to SSR-1305 Transfer Line |  |  |
| GLR-1303 to ANF-1301 Transfer Line |  |  |
| ANF-1301 to LF-1302 Transfer Line |  |  |
| LF-1302 to ANFD-1301 Transfer Line |  |  |
| Associated Flexible Hoses / Jumpers |  |  |
| Valves, Bends and Elbows |  |  |
| White Wipe Cloth Observation |  |  |
| Remarks (if any) |  |  |

# MANUFACTURING AREA CLEANING VERIFICATION

Following completion of equipment cleaning, the associated manufacturing area shall be cleaned as per the approved area cleaning procedure. The cleanliness of the manufacturing area surrounding each equipment shall be verified by visual inspection and white wipe cloth assessment. The inspection shall include the floor, adjacent wall, ceiling, AHU grills/diffusers, and equipment external surface to verify the absence of visible residues and extraneous matter.

A clean, dry, lint-free white wipe cloth shall be used to wipe representative area surfaces. The wipe cloth shall be visually examined for any evidence of product residues, discoloration, stains, fibers, black particles, or other extraneous matter. The observations made by Production and QA personnel shall be recorded in the following table.

Acceptance Criteria:

- No visible product residue, dust, stain, fibers, black particles, or extraneous matter shall be observed.

- The white wipe cloth shall remain visually clean without evidence of residue or discoloration.

- All inspected areas shall comply with established visual cleanliness requirements.

Manufacturing Area Cleaning Verification Summary

| ### Equipment ID | ### Verified by | ### Floor Surrounding Equipment | ### Wall Adjacent to Equipment | ### Ceiling Above Equipment | ### AHU Grills / Diffusers | ### Equipment External Surface | ### Wipe Cloth Observation (Production / QA) | ### Overall Result | ### Sign & date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ### MV-1304 | ### Production |  |  |  |  |  |  |  |  |
|  | ### QA |  |  |  |  |  |  |  |  |
| ### MV-1305 | ### Production |  |  |  |  |  |  |  |  |
|  | ### QA |  |  |  |  |  |  |  |  |
| ### PFR-1301 | ### Production |  |  |  |  |  |  |  |  |
|  | ### QA |  |  |  |  |  |  |  |  |
| ### GLR-1305 | ### Production |  |  |  |  |  |  |  |  |
|  | ### QA |  |  |  |  |  |  |  |  |
| ### GLR-1304 | ### Production |  |  |  |  |  |  |  |  |
|  | ### QA |  |  |  |  |  |  |  |  |
| ### GLR-1303 | ### Production |  |  |  |  |  |  |  |  |
|  | ### QA |  |  |  |  |  |  |  |  |
| ### SSR-1305 | ### Production |  |  |  |  |  |  |  |  |
|  | ### QA |  |  |  |  |  |  |  |  |
| ### ANF-1301 | ### Production |  |  |  |  |  |  |  |  |
|  | ### QA |  |  |  |  |  |  |  |  |
| ### LF-1302 | ### Production |  |  |  |  |  |  |  |  |
|  | ### QA |  |  |  |  |  |  |  |  |
| ### ANFD-1301 | ### Production |  |  |  |  |  |  |  |  |
|  | ### QA |  |  |  |  |  |  |  |  |

# OVERALL CLEANING RESULTS SUMMARY:

Consolidated equipment-wise cleaning validation summary. Result fields are intentionally left blank for recording during report finalization.

| Sr. No. | Equipment ID | Visual Inspection | Swab Samples | Rinse Sample | Extraneous Matter | Nitrosamine | PGI | Visual inspection of Mfg. area | Overall Status | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | MV-1304 |  |  |  |  | NA | NA |  |  |  |
| 2 | MV-1305 |  |  |  |  | NA | NA |  |  |  |
| 3 | PFR-1301 | NA | NA |  |  |  |  |  |  |  |
| 4 | GLR-1305 |  |  |  |  |  |  |  |  |  |
| 5 | GLR-1304 |  |  |  |  |  |  |  |  |  |
| 6 | GLR-1303 |  |  |  |  |  |  |  |  |  |
| 7 | SSR-1305 |  |  |  |  |  |  |  |  |  |
| 8 | ANF-1301 |  |  |  |  | NA | NA |  |  |  |
| 9 | LF-1302 |  |  |  |  | NA | NA |  |  |  |
| 10 | ANFD-1301 |  |  |  |  |  |  |  |  |  |

Acceptance Criteria Summary

| Parameter | Acceptance Criteria |
| --- | --- |
| Visual Inspection | No visible residue, stain, foreign matter, or product traces |
| Product Residue in Swab/Rinse Samples | NMT 10 ppm |
| Extraneous Matter | No black particles and no fibers observed |
| Nitrosamine Impurities | NMT 0.01 ppm each |
| Potential Genotoxic Impurities (PGIs) | NMT 0.2 ppm each |
| Overall Compliance | All applicable acceptance criteria shall be met |

Overall Conclusion:

The cleaning verification study shall be considered acceptable when all applicable visual inspection, swab, rinse, extraneous matter, nitrosamine, PGI results and manufacturing area cleaning visual inspection results comply with predefined acceptance criteria.

# TESTING PROCEDURE

Swab and rinse samples collected during the cleaning validation study shall be analyzed using approved specifications and validated analytical methods to verify the effectiveness of the cleaning procedure. Product residue, extraneous matter, pH, and potential genotoxic impurities (PGIs), as applicable, shall be evaluated. The applicable analytical methods, limits of quantitation (LOQ), limits of detection (LOD), and swab recovery factors are summarized in following table.

| Test parameter | Specification No. | Testing Method No. | LOQ | LOD | Swab recovery |
| --- | --- | --- | --- | --- | --- |
| ISM3 content for rinse and swab samples | QCD-CLS-ISM3-001-00 | QCD-CLT-ISMC-001-00 | 5 ppm | 2 ppm | 85% |
| ISM4A content for rinse and swab samples | QCD-CLS-ISM4A-001-00 | QCD-CLT-ISMC-001-00 | 5 ppm | 2 ppm | 85% |
| ISM4 content for rinse and swab samples | QCD-CLS-ISM-001-00 | QCD-CLT-ISM-001-00 | 1 ppm | 0.3 ppm | 85% |
| Extraneous matter for rinse samples | NA | QCD-GTP-001-00 | NA | NA | NA |
| pH for rinse samples | QCD-SPC-POW-001-01 | QCD-SPT-POW-001-01 | NA | NA | NA |
| Potential genotoxic impurities testing for rinse samples |  |  |  |  |  |
| O-Nitro Toluene | NA | X-PR-IMN-FP-26-006 | 1.5 ppm | 0.5 ppm | NA |
| P-Nitro Toluene | NA | X-PR-IMN-FP-26-006 | 1.5 ppm | 0.5 ppm | NA |
| Mesityl oxide | NA | Annexure-x | 1.5 ppm | 0.5 ppm | NA |

# STATUS OF CLEANING ANALYTICAL METHOD VALIDATION

Analytical methods used for the determination of product residues, nitrosamines, and potential genotoxic impurities in cleaning samples shall be validated or qualified for their intended use. The status of the applicable analytical method validation activities is summarized in the following table.

| Test parameter | Testing Method No. | Method validation protocol No. | Approved date | Method validation Report No. | Approved date | Status |
| --- | --- | --- | --- | --- | --- | --- |
| ISM3 content for rinse and swab samples | QCD-CLT-ISMC-001-00 | NA | NA | IMN-VAL-FP-CLM-25-015 | 15-10-2025 | Completed |
| ISM4A content for rinse and swab samples | QCD-CLT-ISMC-001-00 |  |  |  |  |  |
| ISM4 content for rinse and swab samples | QCD-CLT-ISM-001-00 | PR-MV-26-0011 | 22-09-2026 | NA | NA | Under progress |
| Nitrosamines (NDMA, NMBA, NDEA, NEIPA, NDIPA, NMPA & NDBA) | - | X-VAL-IMN-NSA-25-004 | 26-08-2025 | IMN-VAL-FP-NSA-2-012 | 15-10-2025 | Completed |
| Mesityl Oxide |  |  | - | - | - |  |
| O-Nitro Toluene & P-Nitro Toluene | - | NA | NA | X-PR-IMN-FP-26-006 | 18-03-2026 | Completed |

# EVALUATION OF RESULTS AND REPORTING

  - The cleaning procedure shall be considered verified when all cleaning results comply with the visual inspection, swab, rinse, extraneous matter, pH (wherever applicable), nitrosamine, potential genotoxic impurities acceptance criteria defined in this protocol.

  - Results below the LOQ shall be reported as "Less than LOQ" and results below the LOD as "Not detected", along with the LOQ / LOD values.

  - Any result exceeding the acceptance criteria shall be investigated as per the OOS / deviation SOP. The equipment shall be re-cleaned and re-sampled, and the run shall not be counted as a successful run unless the investigation concludes otherwise with QA approval.

  - A cleaning verification report shall be prepared including the cleaning records, sampling details, analytical results with chromatograms, deviations, conclusion and recommendations, and shall be approved by QA.

# DEVIATIONS

Any deviation observed during execution of this protocol shall be recorded, investigated and closed as per the deviation management SOP, with an impact assessment on the effectiveness of the cleaning procedure and appropriate corrective and preventive actions (CAPA). All deviations shall be summarized in the cleaning verification report.

# REVALIDATION

Revalidation of the cleaning procedure shall be performed whenever changes occur that may impact the effectiveness of the validated cleaning process. Such changes include, but are not limited to, modifications to cleaning agents or solvents, equipment chain, manufacturing process, introduction of new products, failure to meet established acceptance criteria, or any other change with the potential to affect equipment cleanability and product quality. The extent of revalidation shall be determined through the change control and quality risk management process.

# ABBREVIATIONS

| Abbreviation | Description |
| --- | --- |
| P & ID | Process & Instrumentation Diagram |
| GLR | Glass Lined Reactor |
| SSR | Stainless Steel Reactor |
| mg | Milli Gram |
| ml | Milli Liter |
| CVRP | Cleaning Verification Protocol |
| ppm | Parts Per million |
| NDBA | N-Nitrosodibutylamine |
| NDEA | N-Nitrosodiethylamine |
| NDIPA | N-Nitrosodiisopropylamine |
| NDMA | N-Nitrosodimethylamine |
| NEIPA | N-Nitrosoethylisopropylamine |
| NMBA | N-Nitroso-N-methyl-4-aminobutyric acid |
| NMPA | N-Nitrosomethylphenylamine |
| API | Active Pharmaceutical Ingredient |
| BCR | Batch Cleaning Record |
| CAPA | Corrective and Preventive Action |
| DQ / IQ | Design Qualification / Installation Qualification |
| OQ / PQ | Operational Qualification / Performance Qualification |
| FID | Flame Ionization Detector |
| GC | Gas Chromatography |
| GC-MS | Gas Chromatography – Mass Spectrometry |
| HS-GC | Headspace Gas Chromatography |
| LOD | Limit of Detection |
| LOQ | Limit of Quantitation |
| MACO | Maximum Allowable Carryover |
| MBS | Minimum Batch Size |
| MSGL | Mild Steel Glass Lined |
| NLT / NMT | Not Less Than / Not More Than |
| OOS | Out of Specification |
| PDE | Permitted Daily Exposure |
| PGI | Potential Genotoxic Impurity |
| RSD | Relative Standard Deviation |
| SAF | Solvent Adherence Factor |
| STP | Standard Testing Procedure |
| TBU | To Be Updated |
| TDD | Therapeutic Daily Dose |
| TRF | Test Request Form |
| TTC | Threshold of Toxicological Concern |
| URS | User Requirement Specification |
| VMP | Validation Master Plan |

# RELATED DOCUMENTS

| S. No. | Document Title | Document Number |
| --- | --- | --- |
| 1 | Batch cleaning records of ISM4 | To be assigned |
| 2 | Change control | PCC-PRD-26-168 |
| 3 | Cleaning Procedure Development Report | CPDR-PB2-001 |
| 4 | Process Development Report | PDR-ISM-001-03 |
| 5 | Equipment qualification reports (IQ / OQ / PQ) of the equipment train | Refer Section 7.0 |

# LIST OF ANNEXURES

| S. No. | Document Title | Document Number |
| --- | --- | --- |
| 1 | Surface area calculation for process lines | Annexure-1 |
| 2 | PDE report | Annexure-2 |
| 3 | Reference guideline for TDD value | Annexure-3 |
| 4 | Cleaning process flow chart of ISM4 manufacturing equipment | Annexure-4 |

# HISTORY OF THE DOCUMENT

| Version number | Effective date | Reason for revision |
| --- | --- | --- |
| 00 |  | New document |

Upon completion of water reflux, the reflux water shall be completely drained and the reactor shall be dried. The reactor shall then be rinsed with acetone, and the acetone rinse sample shall be collected and submitted for analysis to evaluate the effectiveness of the water reflux cleaning step.

Upon completion of water reflux, the reflux water shall be completely drained and the reactor shall be dried. The reactor shall then be rinsed with acetone, and the acetone rinse sample shall be collected and submitted for analysis to evaluate the effectiveness of the water reflux cleaning step.