| Date: | 18/07/2025 | Deviation No. | DEV/EU/25/004 |
| --- | --- | --- | --- |
| Investigation tool used: 6M 5 Why Brainstorming |  |  |  |
| Other Tools (If any): |  |  |  |
| Not applicable |  |  |  |
| Define: |  |  |  |
| Following checks shall be considered while writing the “Define” section. - Clearly define what happens actually. - Explain what is different than expected. - Mention the location where the deviation has occurred. - Date/time of deviation occurrence and date/time of detection. - Mention the name of personnel who is involved in the deviation. - Mention initial scope of deviation (i.e., impacted product/Material/Equipment/System/Batches/etc.) On 19/06/2025, during the periodic requalification of the Holding Vessel WFI Header Line (as per Protocol No. PRQP-25-PR-002), the Sterilization-in-Place (SIP) cycle was aborted during the sterilization phase after 12 minutes of operation. As per Annexure VII of the Performance Requalification Protocol, the required holding time is 15 minutes. Since the SIP cycle did not complete as per the predefined parameters, a deviation was raised to investigate and identify the root cause. The SIP cycle failed due to a drop in pure steam line pressure, which resulted in inability to maintain the required temperature during the hold phase. The incident occurred in the Production/Filtration Room (GF-88) at 02:31 hrs. and was identified at 03:08 hrs. on the same date by a production staff member (Employee ID: 549). Following the cycle, the exposed Biological Indicator (BI) was submitted to the Microbiology Department for decontamination. Based on the nature of the event, the deviation was assessed, and it was concluded that there is no impact on the vessel’s operational integrity, as the non-conformance was solely due to the temporary pressure drop in the pure steam supply |  |  |  |
| Details Investigation: |  |  |  |
| Measure: Following checks shall be considered while writing the “Measure” section. - Does the summary provide relevant facts and data/information that was reviewed including: environment, process/product history, personnel info (title and job title), controls limits, etc. - Is a summary of the analysis of the factors and data provided? - Is a conclusion statement of the analysis and review provided? - If there were Regulatory Notification, were details provided? - Is the report written in a logical flow and easily understood by the reader? On 19/06/2025, during the periodic requalification of the Holding Vessel WFI Header Line (as per Protocol No. PRQP-25-PR-002), the Sterilization-in-Place (SIP) cycle was aborted during the sterilization phase after 12 minutes of operation. As per Annexure VII of the Performance Requalification Protocol, the required holding time is 15 minutes. Since the SIP cycle did not complete as per the predefined parameters, a deviation was raised to investigate and identify the root cause. Upon initiation of the deviation by the production colleague (Employee ID: 536), the Engineering and Utilities Department took immediate corrective action by attending to the leakage observed in the pure steam generation system. The system was manually stopped to prevent further leakage, which could have posed a potential safety risk. A preliminary risk assessment was performed using the defined risk assessment criteria, and the resulting risk score was 06. Based on this evaluation, the deviation was categorized as Minor. In addition, awareness training regarding this deviation was imparted to the concerned personnel. Emphasis was placed on the critical instruction that pure steam generation must not be stopped if a SIP cycle or related usage is ongoing, to avoid process disruption and non-compliance. The process report for the Pure Steam Generation and Distribution System (Attachment No: I) was reviewed to assess the pressure trend of pure steam. It was observed that the pure steam pressure began to decrease at 02:25 hrs. on 19/06/2025, dropping gradually from 3.6 bar to 0.0 bar by 02:39 hrs. The audit trail report for SCADA system (Attachment No: II) was verified and found that pure steam generation system was stopped on 19/06/2025 at 02:23 hrs. by engineering colleague having employee ID:700310 and further it was started at 02:51 hrs. and pure steam pressure resumed at 02:53 hrs. The SIP report for the Holding WFI Header Line (Attachment No: III) was also verified. It was found that the temperature started decreasing from 02:31 Hrs. eventually falling below 118°C, which is the sterilization abort threshold temperature as defined in the SIP Recipe Parameters (Attachment No: IV). After completion of cycle the exposed biological indicator was submitted to the microbiology department for decontamination Refer to Attachment No: V for the decontamination cycle report. Deviation no. DEV/EU/25/004 raised for same. Upon logging the deviation as an immediate action engineering colleague (Employee ID: 1015) given awareness training to all concern persons. After assuring that pure stem pressure is resumed, the qualification cycle for the Holding WFI Header Line re started on dated 22/06/2025. After completion of cycle verified the reports and results found satisfactory (Attachment No: VI). Hence there is no impact anticipated on equipment operation. As the deviation pertains to the failure of the sterilization cycle during periodic requalification of the WFI header line in the Production Department, and no batch or product impact occurred, a regulatory notification is not required. A personnel interview was conducted with the Engineering Department colleague (Employee ID: 700310), who was the shift operator responsible for the Water System, including the operation of the Pure Steam Generation and Distribution System. Based on the interview (Attachment No. X) findings, it was concluded that the system was manually stopped due to heavy leakage observed from a TC (Tri-Clamp) connection; in order to prevent further damage or a potential safety incident and concern person has trained on the concern SOP No. SOP/DP/EU/014 “Operation of pure steam generation & pure steam distribution system” with revision No. R02. Additionally ADKOM tool was used to identify the root cause for the subjected non-conformance (Attachment No. XI) The damaged gasket from the leakage point was inspected (Attachment No. VII) and verified for its Material of Construction (MOC). It was confirmed that the gasket was made of silicone, which had deteriorated over time, leading to failure during operation. The older work order data was reviewed only one work order was received (Work Order No. WO/EU/24-024) which is related to pressure drop for pure steam generation and distribution system but there is no similar incident was observed (Attachment No. VIII) The PM checklist for pure steam generation was verified (Attachment No. IX) and found that last PM was done on 25/12/2024 and during PM no leakages was identified and there are no checks for verification of gasket condition. Top of Form Conclusion: On 19/06/2025 at 02:23 Hrs. the Pure Steam Generation System was manually stopped by an Engineering colleague (Employee ID: 700310) due to a leakage observed in the system. This action led to a drop in pure steam pressure, which subsequently caused a decrease in the sterilization hold temperature of the WFI Header Line. As a result, the SIP cycle was aborted, and the cycle outcome was recorded as failed. The Biological Indicator (BI) used during the cycle was discarded as per the established decontamination procedure in the Microbiology Department. Following resolution of the issue, the SIP cycle was repeated, and the test results were found to be within acceptable limits, indicating a successful sterilization. Based on personnel interviews and a thorough review of system-related data, the root cause of the leakage was identified as a damaged Tri-Clamp (TC) gasket. Over time, the gasket had deteriorated due to routine operational stress and thermal exposure, eventually leading to a failure in maintaining a proper seal. This resulted in visible leakage from the connection point. Upon noticing the issue, the operator promptly stopped the system as a precautionary measure to prevent any potential escalation or major incidentBottom of Form |  |  |  |
| Analyze: Various investigation tools used to analyze the root cause like 6M, 5 Why, Personnel interview, Brainstorming etc. |  |  |  |
| 6 M Method (If Applicable): - Man: Not Applicable. - Machine: Not Applicable. - Measurement: Not Applicable. - Material: Not Applicable. - Method: Not Applicable. - Milieu (Environment): Not Applicable. Conclusion Not Applicable |  |  |  |
| Note: No 6 M question shall be deleted in the investigation, If any question is not applicable, then mention in ans. as “Not Applicable”. |  |  |  |
| 5 Why Approach (If Applicable): 1. WHY: Why the Sterilization-in-Place (SIP) cycle was aborted during the sterilization phase? Ans. The sterilization cycle was aborted due to a decrease in temperature during the hold phase. 2. WHY: Why temperature was decreases during the sterilization hold time? Ans. As the pure steam pressure decreased, the temperature of the WFI header line also declined, resulting in the failure to maintain sterilization conditions. 3. WHY: Why pure steam pressure was decreases? Ans. Due to stoppage of pure steam generation system, the pure steam pressure decreases. 4. WHY: Why pure steam generation system stopped? Ans. Due to leakage observed on the pure steam generation system, system was stopped immediately. 5. WHY: Why leakage observed in the pure steam generation system. Ans. Due to torn gasket, leakage observed in pure steam generation system. Conclusion: Due to torn/damage gasket, leakage found in pure steam generation system hence as an immediate action system was stopped by water system operator which leads to the failure of ongoing sterilization cycle at production department *Note: Less or more than 5 questions may be asked based on the outcome of earlier questions. |  |  |  |
| Brainstorming: Not Applicable |  |  |  |
| Other Tool if Any: Not Applicable |  |  |  |
| Other Tool if Any: Not Applicable |  |  |  |
| Investigation Outcome: |  |  |  |
| On 19/06/2025, during the periodic requalification of the Holding Vessel WFI Header Line (as per Protocol No. PRQP-25-PR-002), the Sterilization-in-Place (SIP) cycle was aborted during the sterilization phase after 12 minutes of operation. As per Annexure VII of the protocol, a minimum holding time of 15 minutes is required. Since the SIP cycle did not meet the predefined parameters, a deviation was raised for investigation. The incident occurred at 02:31 hrs. in the Production/Filtration Room (GF-88), and was identified by a production colleague (Employee ID:549) at 03:08 hrs. The exposed Biological Indicator (BI) used during the aborted cycle was submitted to the Microbiology Department for decontamination as per standard procedure. A preliminary risk assessment was performed based on the site’s risk matrix. The deviation was categorized as Minor, with a risk score of 06, since there was no impact on the product, equipment performance, or patient safety. As part of the preventive actions, awareness training was conducted for relevant engineering personnel. The training emphasized that the pure steam generation system must not be manually stopped during an active SIP cycle unless absolutely necessary and with proper escalation. Additionally, the importance of early detection of gasket wear or signs of leakage was reinforced. Upon investigation, it was found that the pure steam generation system was manually stopped at 02:23 hrs by an Engineering colleague (Employee ID: 700310) due to significant leakage observed at a Tri-Clamp (TC) connection in the system. The leakage led to a drop in pure steam pressure, resulting in a failure to maintain the required sterilization temperature during the SIP hold phase, thus triggering the abort. Further inspection confirmed that the leakage was caused by a damaged silicone TC gasket, which had deteriorated over time due to operational and thermal stress. This damage compromised the sealing integrity of the connection, causing steam loss. Based on the Why-Why Analysis, it was concluded that: Due to a torn/damaged gasket, leakage occurred in the pure steam generation system. As an immediate safety action, the system was stopped by the water system operator, which directly led to the failure of the ongoing sterilization cycle in the Production Department. After replacement of the damaged gasket and restoration of the system, the SIP cycle was re-initiated. The requalification cycle was successfully completed, and all critical parameters met acceptance criteria. The deviation was thoroughly assessed and concluded to have no impact on the operational integrity of the equipment, as the non-conformance was solely due to a temporary utility disturbance and not due to equipment or process failure. Hence, the regulatory notification is not required. |  |  |  |
| Identified Root Cause/ Probable Cause: |  |  |  |
| - Based on the detailed investigation, most probable root cause as mentioned below - Primary Root Cause Level 1: Equipment / Instrument. Secondary Root Cause Level 2: Mechanical Third Root Cause Level 3: Not Applicable. |  |  |  |
| Impact Assessment (System/ Document/ Product/ Equipment/Patient safety/Past batches): |  |  |  |
| System: The failure was due to a utility issue (pure steam supply) and not equipment malfunction. Post-resolution, the SIP cycle was successfully repeated and passed. Hence, no adverse impact on equipment performance. Document: The deviation occurred during the periodic requalification of the WFI header line and was properly documented through a deviation record and supporting attachments (e.g., SIP reports, process trends, audit trails, interview records). The event was investigated, recorded, and justified in accordance with applicable quality procedures. The only affected document is the SIP qualification record, which initially showed a failed cycle. This has been addressed by repeating the cycle, and the requalification was successfully completed with a passing result. All relevant documents, including SIP cycle reports, BI decontamination records, and the deviation investigation, have been updated and maintained as part of GMP documentation. Product: No product or batch was under processing or exposure during the SIP cycle failure. Therefore, there is no impact on product quality or safety. Equipment: failure was due to a utility issue (pure steam supply) and not equipment malfunction. Post-resolution, the SIP cycle was successfully repeated and passed. Hence, no adverse impact on equipment performance. Patient safety / Past Batches: The deviation did not affect any commercial or clinical batches. No product was exposed to the WFI header line during or prior to the event. Therefore, past batches remain unaffected. |  |  |  |
| Improve: Improve section covers the corrective actions Following checkpoint shall be considered while finalizing the corrective actions, - Were specific corrective Actions identified (including applicable Immediate Actions) to remediate the current issue or situation such that the associated system was returned to a state of control/compliance? - Were specific Corrective Actions identified for each root cause /substantiated probable root cause, as applicable? - Was the Corrective action assigned a unique number, responsible person and due date so it can be tracked? Does the action describe what will be the expected outcome they can be verified? - Was Effectiveness Verification required or not and the rationale for either documented based within the quality impact of the deviation after the investigation concluded? - Are the identified corrective actions achievable based on the information provided? |  |  |  |
| Corrective Action: |  |  |  |
| The investigation has been driven through 5 Why, Personal interview tools. During the investigation, personal interviews was carried out. Based on the detailed investigation, it has been concluded that, The damaged silicone gasket identified as the root cause of the leakage in the pure steam generation system shall be replaced with a PTFE gasket, which offers improved thermal and mechanical durability under steam sterilization conditions. This replacement will help to minimize the risk of gasket failure during operation and ensure reliable performance of the pure steam system. Apart from this other system i.e., Water for Injection system gasket shall be replace with new one (PTFE gasket). |  |  |  |
| Control: Control section covers the preventive actions Following checkpoint shall be considered while finalizing the preventive actions, - Were specific Preventive Actions identified for each root cause /substantiated probable root cause as applicable? - Was the Preventive Action linked the classification of the root cause and explanation given for how it will prevent recurrence? - Was the Preventive action assigned a unique number, responsible person and due date so it can be tracked? Does the action describe and expected outcome can be verified? - Was Effectiveness Verification required or not and the rationale for either documented based on the quality impact of the incident after the investigation concluded? - Was an Interim Plan needed to ensure a state the control while the Preventive Actions were implemented? - Was rationale provided when no Preventive Action were identified? - Does the Final Comments section include rotational to support the conclusion of the investigation and CAPA. - Was each of the impact assessment fields completed correctly - Regulatory Impact, Regulatory notification Product Quality, Validation, Stability, Market/Clinical? - Does the Recommended Lot disposition match the conclusions of the investigation and Impact assessment? - Does the Conclusion include final decision and rationale other regulatory notification required? - CAPA required to release material or batches have been verified to be complete and closed prior to material or batch disposition. Any related CAPA that remain open, but have no impact on the material or batch release and a documented explanation is provided. - Does the Conclusion include a brief summary of root cause, final scope/impact including rationale for product/material disposition, impact assessment and relevant lot/material/equipment details? - Are the identified preventive actions achievable based on the information provided? |  |  |  |
| Preventive Action: |  |  |  |
| The investigation has been carried out through 5 Why and Personal interview during the investigation probable root cause was identified. As per outcome of the investigation following preventive action has been identified. All gaskets used in the pure steam generation and distribution system shall be verified during scheduled preventive maintenance (PM) activities. Gaskets showing signs of wear, deformation, or material degradation shall be replaced based on their physical condition, irrespective of their service duration, to prevent recurrence of leakage and ensure consistent system performance. Additionally, the Preventive Maintenance checklist (Checklist No: PMC/EU/006 & PMC/EU/007) shall be revised to incorporate specific inspection points for gasket condition assessment and replacement criteria. CAPA form shall be initiated to track the preventive action with unique number. Responsibility and target completion date of identified preventive actions shall be assigned in the CAPA form. Interim Action: Awareness training has been given to concern person hence interim action is not required. |  |  |  |
| Document Reviewed: |  |  |  |
| - Review of PM checklist of pure steam generation and distribution system. Checklist No. PMC/EU/006 - Review of Pure steam generation system operation report. - Review of audit trail of SCADA system. - Review of SIP cycle data - Review of Holding WFI header line SIP parameters |  |  |  |
| List of attachment (If applicable): |  |  |  |
| - Attachment No. I: Process report of pure steam generation and distribution system. - Attachment No. II: Audit trial report of SCADA system. - Attachment No. III: SIP report for the holding WFI header line. - Attachment No. IV: SIP recipe parameters - Attachment No. V: Decontamination cycle report - Attachment No.VI: Holding WFI Header Line SIP report - Attachment No. VII: Damaged gasket photo - Attachment No. VIII: Work order history data - Attachment No. IX: PM checklist for pure steam generation and distribution system - Attachment No. X: Personal Interview sheet. - Attachment No. XI: Root cause categorization. - Attachment No. XII: ADKOM Tool |  |  |  |

| Prepared By (Sign/Date) | Reviewed By (Sign/Date) | Reviewed By (Sign/Date) | Reviewed By (Sign/Date) | Reviewed By (Sign/Date | Approved By QA (Sign/Date) |
| --- | --- | --- | --- | --- | --- |